UNDERGRADUATE CAPSTONE RESEARCH & ENGINEERING MANUAL SMART MOBILE-BASED SOLAR-POWERED CHICKEN FEEDING SYSTEM WITH AUTOMATED DISPENSING AND REAL-TIME ALERTS Comprehensive Technical Documentation, System Architecture, Setup Guide, and Non-Programmer User Manual Author & Developer; Jehosue (Jehooooo). System Release Version; 1.0.0+1 (Production MVP Release). Embedded Firmware; ESP32-WROOM-32 DevKit V1 (C++ / FreeRTOS / PlatformIO). Mobile Client; Flutter 3.x (Android Native APK & Safari PWA). Cloud Backend; Google Firebase Realtime Database (asia-southeast1). Official Repository; https://github.com/Jehooooo/SMART-MOBILE-BASED-SOLAR-POWERED-CHICKEN-FEEDING-SYSTEM. Document Date; October 2026.

# Table of Contents

Below is the complete structural index of this technical documentation. In Microsoft Word, press F9 or right-click to update the automated page numbering fields.

| Quick Glossary for Non-Programmers | Terminology reference for poultry operators |
| --- | --- |
| Implementation & Verification Status Legend | Academic classification badges (IMPLEMENTED, TESTED, etc.) |
| Chapter 1: Project Overview | Purpose, objectives, scope, limitations, system capabilities & tech stack |
| Chapter 2: System Architecture and Components | Physical hardware layer, ESP32 GPIO pinout, communication protocols |
| Chapter 3: Complete System Flowcharts | Core workflows (Overall, App, Automated, Firebase, Wi-Fi, Error) |
| Chapter 4: Firebase Configuration Guide | 12-step beginner tutorial, security rules, schema & common errors |
| Chapter 5: Frontend and Backend Communication | 10 end-to-end traced operations with triggers, payloads & files |
| Chapter 6: ESP32 and Machine Configuration | VS Code & PlatformIO setup, power safety warnings, flashing guide |
| Chapter 7: Physical Machine and Mobile Integration | Mechanical assembly, Wi-Fi dual-mode behavior & 9-check checklist |
| Chapter 8: Android Installation and User Guide | APK download, unknown source install, operation guide & iOS PWA |
| Chapter 9: Application Feature User Manual | Every implemented UI screen, controls, expected results & screenshots |
| Chapter 10: Troubleshooting Guide | 14 comprehensive problems, root causes & verified step-by-step solutions |
| Chapter 11: Maintenance and Safety | Calibration cadence, cleaning, battery/solar care, shutdown & stop procedures |
| Chapter 12: Testing and Validation | Software unit/socket suite, compile checks, and master hardware test matrix |
| Chapter 13: Configuration Reference | Tracked file tree, config.h constants, SoftAP REST endpoints & commands |
| Chapter 14: Frequently Asked Questions (FAQ) | 12 authoritative answers grounded strictly in repository code |

Master Technical Documentation, Non-Programmer Setup Guide, and Operational User Manual

Official Repository: https://github.com/Jehooooo/SMART-MOBILE-BASED-SOLAR-POWERED-CHICKEN-FEEDING-SYSTEM

Application Version: 1.0.0+1 (com.smartfeeder.smart_chicken_feeder)

Controller Target: ESP32-WROOM-32 (esp32dev, Arduino / PlatformIO Framework)

Cloud Backend: Google Firebase Authentication & Firebase Realtime Database (asia-southeast1)

Companion Flowchart Suite: docs/SYSTEM_FLOWCHART.md

# Quick Glossary for Non-Programmers

If you do not have a programming or electronics background, use this plain-language reference whenever technical terms appear:

| Term | Plain-English Meaning |
| --- | --- |
| APK (`.apk` file) | An installation file for Android phones (similar to an .exe installer on Windows). Does not work on iPhones. |
| Buck Converter (`LM2596`) | A small voltage-step-down board that safely reduces the 12-volt battery power down to 5.0 volts for the electronics and motor. |
| Calibration / Tare | Tare resets the weighing scale to 0.0 grams when the tray is empty. Calibration teaches the scale what a known weight (like a 500g water bottle) feels like so future portions are accurate. |
| ESP32 | The small computer board ("brain") inside the feeder box that reads the sensors, moves the feed gate motor, and talks to Wi-Fi. |
| Firebase Realtime Database | Google's secure cloud storage that acts as a live message board between your phone and the ESP32 over the internet. |
| Firmware | The permanent control program uploaded onto the ESP32 board from a computer via a USB cable. |
| GPIO Pin | One of the numbered metal pins along the edges of the ESP32 board where wires from sensors and motors connect. |
| Heartbeat | A regular "I am alive" timestamp sent by the ESP32 every 10 seconds so the mobile app knows the machine has power and internet. |
| Load Cell & `HX711` | The metal weighing bar (load cell) under the feed tray and its signal amplifier board (HX711) that measure dispensed feed in grams. |
| NVS (Non-Volatile Storage) | Permanent memory inside the ESP32 chip that remembers your schedules and scale calibration even if the battery is unplugged. |
| PWA (Progressive Web App) | A web version of the app that iPhone/iPad users can save directly onto their Home Screen from Safari without using the App Store. |
| RTC (`DS3231`) | A coin-battery-backed digital clock module that keeps accurate time even if farm Wi-Fi and main power are turned off. |
| Servo Motor (`MG996R`) | A motorized arm that rotates to exact angles (0° closed, 90° open) to open and shut the feed chute gate. |
| SoftAP (Direct Wi-Fi Mode) | A built-in Wi-Fi hotspot (SmartFeeder-AP) broadcast directly by the feeder machine so your phone can control it nearby with zero internet. |
| Telemetry | Live sensor measurements sent from the feeder to your phone (feed hopper percentage, battery voltage, tray weight, solar status). |
| Ultrasonic Sensor (`HC-SR04`) | A distance sensor mounted under the top hopper lid that bounces sound waves off the feed pellets to measure how full the container is. |

# Implementation & Verification Status Legend

To guarantee strict academic and engineering transparency, every capability in this manual is classified using six explicit status labels:

`IMPLEMENTED` — Code is written and active in app/ or firmware/.

`TESTED` — Verified by automated software tests (24/24 Flutter tests passing) and firmware compilation (pio run exit code 0).

`PARTIALLY IMPLEMENTED` — Core capability works, with specific platform constraints documented (e.g., iOS native unsigned CI build vs. working Safari PWA).

`NOT IMPLEMENTED` — Intentionally excluded from system scope (e.g., live camera streaming, multi-farm cloud user management, background push server).

`NOT VERIFIED` — Software/firmware logic is implemented, but physical bench/coop testing with live hardware is required to confirm real-world mechanical/electrical metrics.

`RECOMMENDED IMPROVEMENT` — Optional enhancement not currently coded, clearly separated from existing features.

# Chapter 1: Project Overview

## 1.1 Project Introduction

Poultry farming in rural and off-grid agricultural communities faces two persistent operational challenges: inconsistent manual feeding schedules—which lead to feed wastage or uneven flock growth—and unreliable grid electricity or rural internet connectivity.

The Smart Mobile-Based Solar-Powered Chicken Feeding System with Automated Dispensing and Real-Time Alerts solves these problems by combining a 12V solar-charged off-grid power subsystem, an ESP32-WROOM-32 embedded controller with closed-loop load-cell weighing and ultrasonic hopper sensing, Firebase Realtime Database cloud synchronization, and a cross-platform Flutter mobile application. Whether the farmer is miles away using mobile data or standing beside an off-grid coop with no internet at all, the feeder delivers measured rations on schedule and alerts the operator when feed or battery reserves run low.

## 1.2 Purpose and Objectives

Automate Poultry Ration Dispensing: Eliminate missed or late feedings using programmable daily schedules backed by an onboard DS3231 Real-Time Clock (RTC) and non-volatile flash memory (NVS).

Ensure Portion Accuracy: Measure dispensed feed in real time using a 5 kg strain-gauge load cell and HX711 24-bit amplifier, closing the MG996R servo gate automatically once the target gram weight is reached (with timed fallback when operating in duration mode).

Operate Independently of Grid Power: Run 24/7 on a 12V solar panel, solar charge controller, rechargeable 12V battery, and 5.0V LM2596 buck regulator, protected by an 11.2V deep-discharge motor lockout.

Provide Real-Time Remote Monitoring & Alerts: Stream hopper level (%, estimated kg, estimated supply days), battery state (%, Volts, solar charging state), and fault alerts to the user's smartphone.

Guarantee Off-Grid Resilience: Support Dual-Mode Connectivity—switching seamlessly between Firebase Realtime Cloud (remote access from anywhere) and Direct Wi-Fi HTTP SoftAP (SmartFeeder-AP at http://192.168.4.1 with zero internet required).

Support Empirical Research & Thesis Evaluation: Log target weight, actual weight, gram variance, percentage accuracy, battery voltage, and hopper depth for every cycle, with one-tap CSV export formatted for Microsoft Excel and SPSS.

## 1.3 Scope and Limitations

### Within Project Scope

Single-unit smart poultry pellet/crumble feeder controlled by an ESP32-WROOM-32.

Automated daily time-based scheduling (up to 16 concurrent schedules stored on-device in ESP32 NVS).

Remote manual dispensing (1–30 seconds / ~20g–600g from Dashboard) and immediate mid-cycle Emergency Stop.

Real-time ultrasonic hopper level sensing (5.0 cm full to 30.0 cm empty), load-cell tray weighing, and 12V battery voltage telemetry.

Android native APK installation (Android 6.0+ / API 21+) and browser/iOS Safari PWA access for Cloud mode.

Local hardware diagnostics and 2-point scale calibration wizard inside the mobile app.

### System Limitations (Verified Against Implementation)

Wi-Fi Frequency Band: The ESP32 radio supports 2.4 GHz Wi-Fi only (802.11 b/g/n). It cannot connect to 5 GHz-only routers unless a 2.4 GHz band is enabled.

Foreground/Active Stream Notifications (`PARTIALLY IMPLEMENTED`): Local OS notifications (flutter_local_notifications) are evaluated whenever the mobile app is running or streaming telemetry from Firebase/SoftAP. Firebase Cloud Functions / FCM server-push background wakes when the app is completely force-closed are not implemented (kept serverless on the free Firebase Spark plan).

Pellet Consistency Requirement: Mechanical gravity-gate dispensing works best with dry poultry pellets or uniform crumbles; wet mash can bridge inside the hopper and trigger the 6-second flow-stall safety cutoff.

iOS Direct SoftAP Limitation: On iPhones running the HTTPS-hosted Web App (PWA), Apple Safari blocks insecure local HTTP requests (http://192.168.4.1) due to browser Mixed-Content security rules. iOS users can use Firebase Cloud Mode inside the PWA, or open http://192.168.4.1 directly in a new Safari tab when connected to SmartFeeder-AP to use the ESP32's built-in HTML control page.

Single Shared Feeder Namespace by Default: Both the ESP32 firmware and mobile app default to the root database paths (/telemetry, /commands, /schedules, /logs), meaning all phones signed into the same Firebase project monitor and control the same feeder unit.

## 1.4 Main Features & Implementation Matrix

| Feature | Repository Location | Implementation Status | Verification Status |
| --- | --- | --- | --- |
| Live Telemetry Dashboard (Hopper %, kg, days left, Solar status, Battery V/%) | app/lib/screens/dashboard_screen.dart | IMPLEMENTED | TESTED (Software) / NOT VERIFIED (Live Hardware) |
| Closed-Loop Gravimetric Dispensing (HX711 weight feedback + settle read) | firmware/src/main.cpp (dispenseFeed) | IMPLEMENTED | TESTED (Compile) / NOT VERIFIED (Physical Feed) |
| Timed Duration Dispensing (1–60s @ ~20g/s fallback) | firmware/src/main.cpp (dispenseFeedDuration) | IMPLEMENTED | TESTED (Compile) / NOT VERIFIED (Physical Feed) |
| Remote Manual Feed & Emergency Stop | app/lib/services/feeder_service.dart, main.cpp | IMPLEMENTED | TESTED (24/24 Automated Suite) |
| Autonomous Offline Scheduling (DS3231 RTC + NVS lastSchSlot lock) | firmware/src/main.cpp (checkAutonomousSchedules) | IMPLEMENTED | TESTED (Compile) / NOT VERIFIED (Bench Power-Cycle) |
| Dual-Mode Connectivity (Firebase Cloud + SmartFeeder-AP 192.168.4.1) | firebase_hardware_adapter.dart, rest_hardware_adapter.dart, main.cpp | IMPLEMENTED | TESTED (Loopback Socket Tests) |
| Embedded Standalone Web Portal (GET http://192.168.4.1/) | firmware/src/main.cpp (handleRootPortal) | IMPLEMENTED | TESTED (Compile) |
| Safety Interlocks (0° boot close, 11.2V battery lock, 6s stall, 30s timeout, 60s stale cmd purge) | firmware/src/main.cpp, firmware/include/config.h | IMPLEMENTED | TESTED (Code Audit & Compile) |
| 2-Point Scale Calibration & Servo Sweep Suite | app/lib/screens/diagnostics_screen.dart | IMPLEMENTED | TESTED (Loopback Socket Tests) |
| Feeding History Analytics & UTF-8 BOM CSV Export | app/lib/screens/history_screen.dart, file_exporter_io.dart | IMPLEMENTED | TESTED (Software) |
| Local Safety Notifications (15-min cooldown + +2% hysteresis) | app/lib/services/alert_notification_service.dart | IMPLEMENTED | TESTED (Unit Tests) |
| Cloud Server Push When App Force-Killed (FCM Backend Server) | — | NOT IMPLEMENTED | Serverless design (No Cloud Functions) |

## 1.5 System Capabilities

Dispensing Capacity: Configurable hopper capacity preset in app (3.0 kg, 5.0 kg standard, 10.0 kg, or 15.0 kg).

Schedule Capacity: Up to 16 daily routines persisted simultaneously in ESP32 NVS flash, with per-day-of-week bitmask support (1 = Monday through 7 = Sunday).

Telemetry Cadence: Every 10 seconds during standby (TELEMETRY_INTERVAL_MS = 10000), accelerating to every 1.5 seconds during active dispensing (DISPENSE_STREAM_MS = 1500).

Command Poll Latency: ESP32 polls /commands/dispense.json every 1.5 seconds (COMMAND_POLL_MS = 1500) over HTTPS, or responds instantaneously (< 50 ms) in Direct Wi-Fi SoftAP mode.

Heartbeat Freshness Window: Mobile app marks the feeder Online only when lastHeartbeat is less than 3 minutes old (FeederTelemetry.isOnline in app/lib/models/telemetry.dart).

## 1.6 Technologies and Frameworks Used

| Layer | Technology / Package | Version / Target | Purpose |
| --- | --- | --- | --- |
| Mobile & Web UI | Flutter SDK (Dart) | Dart >=3.5.0 <4.0.0 | Cross-platform Android APK & Web PWA with custom Liquid Glass theme |
| Cloud SDKs (App) | firebase_core, firebase_auth, firebase_database | ^4.15.0, ^6.7.0, ^12.6.0 | Native real-time WebSocket database streams and anonymous authentication |
| HTTP & Caching (App) | http, shared_preferences, intl | ^1.6.0, ^2.5.6, ^0.20.3 | Direct SoftAP REST calls, Firebase REST fallback, and offline local cache |
| Notifications (App) | flutter_local_notifications | ^22.3.1 | High-priority Android/iOS local safety alerts |
| Embedded Firmware | PlatformIO + Arduino Framework (espressif32) | C++ (esp32dev) | Non-blocking microcontroller firmware (RAM 15.3%, Flash 53.4%) |
| Firmware Libraries | ESP32Servo, HX711, RTClib, ArduinoJson | ^3.0.5, ^0.7.5, ^2.1.4, ^7.2.0 | PWM gate control, 24-bit load-cell ADC, DS3231 I2C clock, JSON serialization |
| Cloud Backend | Firebase Realtime Database + Auth | asia-southeast1 (Singapore) | Secure (auth != null) cloud state synchronization |
| CI / Release | GitHub Actions (ios_build.yml) | Ubuntu & macOS Runners | Automated Android split-ABI release APK builds & GitHub Releases |

## 1.7 How the Entire System Works (Plain-English Walkthrough)

Powering Up: Sunlight charges a 12V battery through a solar charge controller. An LM2596 buck converter steps that 12V down to a steady 5.0V to run the ESP32 controller and the MG996R gate motor. Immediately on startup, the ESP32 forces the feed gate closed (0°) so feed can never spill accidentally during a reboot.

Sensing the Feeder: Every 10 seconds, the ESP32 checks three things:

How high the feed pellets are inside the hopper (using the HC-SR04 ultrasonic sensor under the lid).

How many grams of feed are sitting on the weighing tray (using the HX711 load cell).

How much charge is left in the 12V battery and whether the solar panel is actively charging it (using GPIO 34 voltage sensing).

Connecting to Your Phone:

With Farm Wi-Fi (Cloud Mode): The ESP32 sends those readings to Google Firebase in the cloud. Your phone reads them from anywhere in the world over Wi-Fi or mobile data (4G/5G).

Without Internet (Off-Grid Mode): The ESP32 also broadcasts its own Wi-Fi signal named `SmartFeeder-AP`. You can connect your Android phone directly to SmartFeeder-AP and switch the app to Direct Wi-Fi HTTP to control the machine standing nearby.

Feeding the Chickens:

Scheduled Feeding: When the onboard DS3231 clock hits a scheduled feeding time (for example, 7:00 AM), the ESP32 checks that battery voltage is safe (>= 11.2V), zeroes the scale, opens the servo gate to 90°, watches the HX711 scale until the target weight (or duration) is dispensed, and snaps the gate shut (0°). Even if Wi-Fi is completely off, scheduled feedings still happen on time!

Manual Feeding: When you tap DISPENSE NOW on your phone, the app sends a command to the ESP32. The ESP32 deletes the queued command before opening the motor (so a sudden power blink can never make it dispense twice), pours the feed while streaming live gram progress to your screen, closes the gate, and saves a permanent accuracy record to History & Logs.

## 1.8 System Architecture Overview

At an architectural level, the repository follows a Decoupled Adapter Pattern (FeederHardwareAdapter in app/lib/services/hardware/hardware_adapter.dart). None of the UI screens talk directly to Firebase or HTTP sockets; instead, every screen observes FeederService, which delegates communication to either FirebaseHardwareAdapter (Cloud) or RestHardwareAdapter (Local SoftAP). On the hardware side, firmware/src/main.cpp runs a deterministic, non-blocking loop() state machine that services both the local port-80 WebServer and HTTPS Firebase REST endpoints concurrently (WIFI_AP_STA).

# Chapter 2: System Architecture and Components

## 2.1 Complete Hardware and Software Architecture Diagram

PROCESS WORKFLOW & ARCHITECTURE DIAGRAM The complete graphical flowchart for this section is rendered in high resolution in docs/SYSTEM_FLOWCHART.md. Below are the sequential operational logic steps: flowchart TB subgraph SolarSub["Power & Regulation Subsystem"] PANEL["12V Solar Panel"] --> CTRL["12V Solar Charge Controller"] CTRL <--> BATT["12V Lead-Acid / LiFePO4 Battery"] BATT --> LM2596["LM2596 Buck Converter (Tuned to 5.0V DC)"] BATT --> DIV_BAT["Battery Divider (40k Ohm High / 10k Ohm Low)"] end subgraph MechSensors["Sensors & Dispensing Mechanism"] US["HC-SR04 Ultrasonic Sensor (Hopper Lid)"] DIV_ECHO["Echo Divider (1k Ohm / 2k Ohm -> 3.3V Safe)"] LC["5kg Strain-Gauge Load Cell (Weighing Tray)"] HX["HX711 24-Bit Load Cell Amplifier"] RTC["DS3231 I2C RTC Module (CR2032 Coin Cell)"] MG["MG996R Metal-Gear Servo Motor (Feed Gate)"] US --> DIV_ECHO LC --> HX end subgraph ESP32Box["ESP32-WROOM-32 Controller (firmware/src/main.cpp)"] MCU["ESP32 DevKit V1 (30-Pin)"] PREFS["NVS Flash ('feeder' Namespace)"] HTTPD["Local WebServer (192.168.4.1:80)"] MCU <--> PREFS MCU --- HTTPD

## 2.2 Component Descriptions and Individual Responsibilities

| Component | Verified Model / Spec | Individual Responsibility in the System |
| --- | --- | --- |
| Mobile Application | Flutter 3.x (smart_chicken_feeder v1.0.0) | Displays real-time hopper, solar, and battery gauges; dispatches manual feed, stop, tare, calibration, and servo test commands; manages daily schedules; fires local safety notifications; exports CSV research logs. |
| Microcontroller | ESP32-WROOM-32 (esp32dev 30-pin DevKit V1) | Reads all sensors, runs safety interlocks, drives the servo PWM gate, maintains NVS schedules, hosts the SmartFeeder-AP local server, and syncs with Firebase over Wi-Fi. |
| Cloud Database | Firebase Realtime Database (asia-southeast1) | Stores live /telemetry, transient /commands/dispense & /commands/stop, /commands/latest status acknowledgements, /schedules, and /logs behind auth != null security rules. |
| Dispensing Motor | MG996R High-Torque Metal-Gear Servo | Mechanically slides or rotates the hopper outlet gate between 0° (SERVO_CLOSED_ANGLE) and 90° (SERVO_OPEN_ANGLE). |
| Weighing Sensor | 5 kg Aluminum Bar Load Cell + HX711 24-bit ADC | Measures exact mass (grams) of feed dispensed into the tray in real time for closed-loop cutoff and empirical accuracy calculation. |
| Hopper Level Sensor | HC-SR04 Ultrasonic Distance Sensor | Measures empty air gap from hopper lid down to pellet surface (5.0 cm = 100% full, 30.0 cm = 0% empty) using a 3-ping average filter. |
| Real-Time Clock | DS3231 Temperature-Compensated I2C RTC + CR2032 | Preserves accurate date and time across power cycles and internet outages so autonomous feeding schedules never drift. |
| Solar Panel | 12V Monocrystalline Solar Panel | Harvests sunlight outdoors and supplies charging current to the solar charge controller during daylight hours. |
| Solar Charge Controller | 12V PWM Solar Charge Controller | Regulates solar panel voltage to prevent overcharging the 12V battery and blocks reverse current drain at night. |
| Storage Battery | 12V Sealed Lead-Acid (SLA) or LiFePO4 Battery | Buffers energy for 24/7 operation, night/rainy-day feeding, and peak servo motor current surges. |
| Voltage Regulator | LM2596 DC-DC Step-Down Buck Converter | Steps 11.2V–14.4V battery bus down to a strictly regulated `5.0V DC` (<= 3A) rail for the MG996R servo and ESP32 5V/VIN supply. |
| Voltage Dividers | 1) 40 kΩ / 10 kΩ (Battery) <br> 2) 1 kΩ / 2 kΩ (Ultrasonic Echo) | 1) Scales 0–16.5V battery voltage by 1/5 (5.0x ratio) to 0–3.3V safe for ESP32 GPIO 34. <br> 2) Scales 5V HC-SR04 ECHO pulse down to 3.3V safe for GPIO 18. |

## 2.3 Verified ESP32 GPIO Pin Assignment Matrix

All pin numbers below are verified directly from `firmware/include/config.h`:

| Subsystem | Constant in `config.h` | Signal | ESP32 Pin | Logic / Supply Voltage | Critical Wiring Note |
| --- | --- | --- | --- | --- | --- |
| MG996R Servo | PIN_SERVO | PWM Control | `GPIO 13` | 3.3V PWM; `5.0V` Buck Power | Never wire Servo VCC to ESP32 3.3V or VIN; wire directly to LM2596 5.0V output! |
| HX711 Scale | PIN_HX711_DOUT | Data Out (DT) | `GPIO 16` | 3.3V Logic (VCC 3.3V/5V) | Keep load-cell analog wires short/braided away from motor leads. |
| HX711 Scale | PIN_HX711_SCK | Serial Clock (SCK) | `GPIO 17` | 3.3V Logic | Driven as digital output by ESP32. |
| HC-SR04 Hopper | PIN_US_TRIG | Trigger (TRIG) | `GPIO 5` | 3.3V Output (VCC = 5.0V) | HC-SR04 VCC requires 5.0V rail for reliable acoustic range. |
| HC-SR04 Hopper | PIN_US_ECHO | Echo (ECHO) | `GPIO 18` | `3.3V` Divided Input | Must pass through 1 kΩ (series) / 2 kΩ (to GND) voltage divider! |
| DS3231 RTC | PIN_RTC_SDA | I2C Data (SDA) | `GPIO 21` | 3.3V I2C | Insert CR2032 backup coin cell into DS3231 holder. |
| DS3231 RTC | PIN_RTC_SCL | I2C Clock (SCL) | `GPIO 22` | 3.3V I2C | Standard ESP32 hardware I2C clock pin. |
| Battery Sense | PIN_BATTERY_ADC | Analog Sense | `GPIO 34` | 0 – 3.3V ADC (11dB atten.) | Input-only ADC1 pin; connect midpoint of 40 kΩ (to +12V) and 10 kΩ (to GND). |
| Status LED | PIN_STATUS_LED | Digital Out | `GPIO 2` | 3.3V Onboard LED | Lights HIGH while dispensing gate is active; LOW when idle. |

## 2.4 Communication Protocols and End-to-End Data Flow

Internal Hardware Buses (ESP32 ↔ Peripherals):

I2C (`Wire` on `GPIO 21/22`): Bidirectional digital communication between ESP32 and DS3231 RTC.

Two-Wire Synchronous Serial (`GPIO 16/17`): Reads 24-bit signed weight conversions from HX711 with non-blocking 250 ms readiness timeout.

50 Hz Servo PWM (`GPIO 13`, `500–2400 µs` pulse width): Commands MG996R angular position (0°–90°).

Ultrasonic Time-of-Flight (`GPIO 5/18`): 10 µs trigger pulse followed by pulseIn(..., 30000) echo duration measurement (distance_cm = duration_us * 0.0343 / 2).

12-Bit Calibrated Millivolt ADC (`GPIO 34`): analogReadMilliVolts(34) averaged over 4 samples and multiplied by BATTERY_DIVIDER_RATIO (5.0).

Cloud Transport (ESP32 ↔ Firebase ↔ Flutter):

ESP32 Side: Uses WiFiClientSecure (TLS/HTTPS) + HTTPClient against Firebase IdentityToolkit REST API (accounts:signInWithPassword or accounts:signUp) to obtain a bearer idToken, then issues authenticated GET, PUT, POST, PATCH, and DELETE JSON requests to https://<FIREBASE_HOST>/*.json?auth=<idToken>.

Flutter Side: Uses native FirebaseDatabase WebSocket listeners (onValue) on Android/iOS/Web for sub-second push updates, with automatic fallback to HTTPS REST polling (http package) if native streams are unavailable.

Off-Grid Direct Transport (Flutter / Browser ↔ ESP32 SoftAP):

HTTP/1.1 JSON REST over Local Wi-Fi (`http://192.168.4.1:80`): Phone joins SmartFeeder-AP (WPA2-PSK), and RestHardwareAdapter polls GET /api/telemetry every 2 seconds (GET /api/logs every 6 seconds) while sending POST /api/* JSON commands directly to the ESP32.

# Chapter 3: Complete System Flowcharts

Note: For the complete 11-diagram flowchart reference (including ESP32 internal loop timers, sensor priority evaluation, and notification latching), see the dedicated companion file `docs/SYSTEM_FLOWCHART.md`. The six core operational flowcharts are presented below.

## 3.1 Overall System Flowchart

Shows the end-to-end relationship between the Mobile Application, Firebase Realtime Database, ESP32 Microcontroller, and Physical Feeder Machine.

PROCESS WORKFLOW & ARCHITECTURE DIAGRAM The complete graphical flowchart for this section is rendered in high resolution in docs/SYSTEM_FLOWCHART.md. Below are the sequential operational logic steps: flowchart LR USER(["Farm Operator / Smartphone"]) <--> APP["Flutter Mobile App (Dashboard, Schedules, History, Diagnostics)"] APP <-->¦"Mode A: Internet / 4G / Wi-Fi"¦ FB[("Firebase Realtime Database (asia-southeast1)")] APP <-->¦"Mode B: Off-Grid Wi-Fi (192.168.4.1)"¦ ESP["ESP32-WROOM-32 Controller"] FB <-->¦"Station Wi-Fi (HTTPS JSON)"¦ ESP subgraph PhysicalMachine["Physical Solar Feeder Machine"] SOLAR["12V Solar + Battery + 5V Buck"] --> ESP SENS["HX711 Scale + HC-SR04 Ultrasonic + DS3231 RTC"] --> ESP ESP --> GATE["MG996R Servo Feed Gate (0 deg / 90 deg)"] end

## 3.2 Mobile Application Flowchart

Covers application startup, initialization, Firebase/SoftAP connection, dashboard loading, sensor retrieval, manual feeding, scheduled feeding, settings, notifications, and error handling.

PROCESS WORKFLOW & ARCHITECTURE DIAGRAM The complete graphical flowchart for this section is rendered in high resolution in docs/SYSTEM_FLOWCHART.md. Below are the sequential operational logic steps: flowchart TD A(["App Launch"]) --> B["Initialize Flutter Binding, SettingsService & AlertNotificationService"] B --> C{"First Run? (!onboardingComplete)"} C -- "Yes" --> D["Show 4-Slide OnboardingScreen -> Mark Complete"] --> E C -- "No" --> E["Initialize Firebase & Selected HardwareAdapter (Cloud or SoftAP)"] E --> F["Load Cached Telemetry, Schedules & Logs from SharedPreferences"] F --> G{"Connection & Heartbeat (< 3 min) Valid?"} G -- "No" --> H["Show ConnectionBanner ('Offline' / 'Retry') & Block Manual Motor Trigger"] G -- "Yes" --> I["Mark Status 'Online' & Stream Live Sensor Readings to Dashboard"] H --> J["Main 4-Tab Navigation Shell"] I --> J J --> K["Dashboard: View Hopper %, Solar/Battery & Trigger Manual Feed (1-30s) or Emergency Stop"] J --> L["Schedules: Add / Edit / Toggle / Test / Delete Daily Routines (1-60s)"] J --> M["History & Logs: View Accuracy Metrics, Filter Logs & Download UTF-8 CSV"] J --> N["Settings & Diagnostics: Switch Cloud/SoftAP, Set Alert Thresholds, Calibrate Scale & Test Servo"] I --> O["AlertNotificationService: Evaluate Faults, Low Feed (<20%) & Low Battery (<20% / <11.5V)"] O --> P["Dispatch High-Priority Local OS Notification (15-Min Cooldown)"]

## 3.3 Automated Feeding Flowchart

Traces all 11 stages of an automated feeding cycle from schedule/command trigger through validation, safety checks, closed-loop weighing, gate shutoff, logging, Firebase sync, and mobile display:

PROCESS WORKFLOW & ARCHITECTURE DIAGRAM The complete graphical flowchart for this section is rendered in high resolution in docs/SYSTEM_FLOWCHART.md. Below are the sequential operational logic steps: flowchart TD S1(["1. Schedule Matches DS3231 Time OR Remote Command Arrives"]) --> S2{"2. Validate Command: Unique ID & Age <= 60s?"} S2 -- "Invalid / Expired" --> REJ["Purge Command & Mark 'failed' ('expired_stale_command')"] S2 -- "Valid" --> DEL_PRE["Pre-Actuation Safety: DELETE /commands/dispense.json & Save ID to NVS"] DEL_PRE --> S3{"3. Check Operational Conditions: Battery >= 11.2V & Pre-Dispense Scale Tare OK?"} S3 -- "Battery < 11.2V" --> F_BAT["Abort: Keep Gate at 0 deg ¦ Record 'battery_cutoff_11v2'"] --> S9 S3 -- "HX711 Tare Failed" --> F_TARE["Abort: Keep Gate at 0 deg ¦ Record 'hx711_tare_failed_pre_dispense'"] --> S9 S3 -- "Conditions Safe" --> S4["4. Activate MG996R Servo Motor -> Open Gate to 90 deg"] S4 --> S5["5. Pellets Flow Into Weighing Tray"] S5 --> S6["6. HX711 Load Cell Measures Dispensed Weight (250ms Non-Blocking Sample)"] S6 --> S6_ERR{"3 Consecutive HX711 Errors?"} S6_ERR -- "Yes" --> F_HX["Emergency Close (0 deg) ¦ 'hx711_mid_dispense_fault'"] --> S9 S6_ERR -- "No" --> S7{"7. dispensedGrams >= targetGrams?"} S7 -- "No" --> SAFETY_LOOP{"Safety Checks: Stall (>6s with <2g), Timeout (>30s), or Emergency Stop?"} SAFETY_LOOP -- "Stall / Timeout / Stop" --> F_SAFE["Immediate Gate Close (0 deg) ¦ Set Fault/Abort Code"] --> S9 SAFETY_LOOP -- "Normal Flow" --> STREAM_15["Push Live Weight Progress Every 1.5s"] --> S5 S7 -- "Yes (Target Reached)" --> S8["8. Close Servo Gate to 0 deg -> Wait 250ms Settling -> Read Final Grams"] S8 --> S9["9. Record FeedingLog (Target, Actual, Duration, Trigger, Pass/Fail, Volts, Hopper %)"] S9 --> S10["10. Push /logs, /telemetry & /commands/latest to Firebase (When Online)"]

## 3.4 Firebase Communication Flowchart

Explains how commands and telemetry flow between the Flutter App, Firebase Realtime Database, and ESP32, including how commands are written, retrieved, deleted pre-actuation, acknowledged, and completed.

PROCESS WORKFLOW & ARCHITECTURE DIAGRAM The complete graphical flowchart for this section is rendered in high resolution in docs/SYSTEM_FLOWCHART.md. Below are the sequential operational logic steps: sequenceDiagram participant App as Flutter App participant RTDB as Firebase RTDB participant ESP as ESP32 Firmware App->>RTDB: 1. Write /commands/dispense (status: "pending") + /commands/latest ESP->>RTDB: 2. Poll GET /commands/dispense.json (Every 1.5s) RTDB-->>ESP: Return JSON Command ESP->>RTDB: 3. DELETE /commands/dispense.json (Before Moving Motor) ESP->>RTDB: 4. PATCH /commands/latest.json (status: "inProgress") RTDB-->>App: Stream Acknowledgement ("Dispensing in Progress...") loop Every 1.5s While Gate Open ESP->>RTDB: PUT /telemetry.json (Live currentWeightGrams) RTDB-->>App: Update Progress Bar end ESP->>RTDB: 5. POST /logs.json + PUT /telemetry.json + PATCH /commands/latest.json (status: "completed", actualGrams) RTDB-->>App: Display Final Result & Accuracy Log

## 3.5 Wi-Fi Connection Flowchart

Shows initial WIFI_AP_STA dual-mode setup, connection attempts, 15-second background reconnection, network failure fallback, and cloud recovery.

PROCESS WORKFLOW & ARCHITECTURE DIAGRAM The complete graphical flowchart for this section is rendered in high resolution in docs/SYSTEM_FLOWCHART.md. Below are the sequential operational logic steps: flowchart TD BOOT(["ESP32 Boot"]) --> AP_ON["Enable WIFI_AP_STA & Broadcast 'SmartFeeder-AP' (192.168.4.1)"] AP_ON --> HAS_CFG{"WIFI_SSID Configured in secrets.h?"} HAS_CFG -- "No" --> LOCAL_ONLY["Run in Off-Grid Mode (SoftAP HTTP + DS3231 RTC Schedules)"] HAS_CFG -- "Yes" --> ATTEMPT["Attempt Station Wi-Fi Connect (Up to 6 Seconds)"] ATTEMPT --> CONN_OK{"Connected to Router?"} CONN_OK -- "Yes" --> INIT_SYNC["Sync NTP Clock + Authenticate Firebase + Purge Old Commands + Sync Schedules"] CONN_OK -- "No" --> CONTINUE_OFF["Do Not Block Boot: Keep SoftAP & RTC Active"] INIT_SYNC --> TIMER_15["Every 15s: Check WiFi.status()"] CONTINUE_OFF --> TIMER_15 TIMER_15 --> IS_UP{"WL_CONNECTED?"} IS_UP -- "Yes" --> CLOUD_OK["Continue 10s Telemetry & 1.5s Command Polling"] IS_UP -- "No" --> RETRY["Call Non-Blocking WiFi.reconnect() (SoftAP & Schedules Unaffected)"] RETRY --> RESTORED{"Reconnected on Subsequent Check?"} RESTORED -- "No" --> TIMER_15 RESTORED -- "Yes" --> RECOVER["Re-Sync NTP + Purge Stale Commands + Sync Schedules"] --> CLOUD_OK

## 3.6 Error Handling Flowchart

Distinguishes currently implemented fault-handling paths in the codebase from recommended future improvements:

PROCESS WORKFLOW & ARCHITECTURE DIAGRAM The complete graphical flowchart for this section is rendered in high resolution in docs/SYSTEM_FLOWCHART.md. Below are the sequential operational logic steps: flowchart TD ERR(["System Fault / Abnormal Event"]) --> TYPE{"Failure Scenario"} TYPE -- "Internet Unavailable" --> E1["[IMPLEMENTED] App shows Offline Banner & cached data; ESP32 runs DS3231 schedules locally & serves SoftAP 192.168.4.1"] TYPE -- "Firebase Unavailable / 401" --> E2["[IMPLEMENTED] ESP32 refreshes Auth token on HTTP 401 & retries next cycle; App falls back to REST / Retry button"] TYPE -- "ESP32 Disconnected" --> E3["[IMPLEMENTED] App detects lastHeartbeat >= 3 min old -> Locks Manual Feed button & warns 'Feeder controller offline'"] TYPE -- "Sensor Failure (HX711 / HC-SR04)" --> E4["[IMPLEMENTED] 250ms HX711 timeout or 3 failed ultrasonic cycles -> Sets sensorError, locks manual feed & fires Critical Alert"] TYPE -- "Servo Jam / Empty Hopper" --> E5["[IMPLEMENTED] < 2.0g gain in 6s or > 30s total -> Closes gate to 0 deg & logs 'dispense_flow_stall' / 'dispense_timeout'"] TYPE -- "Low Battery (< 11.2V)" --> E6["[IMPLEMENTED] Hard motor cutoff blocks servo actuation at < 11.2V to prevent ESP32 brownout"] TYPE -- "Invalid / Stale Command" --> E7["[IMPLEMENTED] Commands > 60s old or matching NVS lastCmdId are rejected & deleted without moving servo"] TYPE -- "Interrupted Dispensing" --> E8["[IMPLEMENTED] Boot immediately shuts gate to 0 deg; pre-deleted command cannot re-trigger"] E5 -. "Not In Current Firmware" .-> R1["[RECOMMENDED IMPROVEMENT] Add physical current-sense feedback or auto-reverse servo unjam wiggle"] E1 -. "Not In Current Firmware" .-> R2["[RECOMMENDED IMPROVEMENT] Buffer offline feeding logs in ESP32 flash to upload once Wi-Fi returns"]

# Chapter 4: Firebase Configuration Guide

## 4.1 Security Rules & Who Needs This Chapter

If you are an ordinary end user installing the pre-built Android APK (v1.0.0) onto your phone to control an already-configured feeder machine, you can skip directly to [Chapter 8](#chapter-8-android-installation-and-user-guide)—Firebase is already compiled into the app!

If you are setting up a brand-new Firebase cloud account for your own copy of the machine (Developer / System Administrator Setup), follow Steps 1 through 12 below.

STRICT SECURITY REQUIREMENT: Never paste real API keys, passwords, or private tokens into public documents or Git commits. Both firmware/include/secrets.h and app/lib/firebase_options.dart are protected by .gitignore. All examples below use safe placeholders such as <YOUR_FIREBASE_API_KEY>.

## 4.2 Step-by-Step Firebase Setup Tutorial (Steps 1–12)

### Step 1: Create a Google Account (If Necessary)

Open your web browser (Google Chrome or Microsoft Edge) and go to https://accounts.google.com/signup.

Complete the form to create a free Google (@gmail.com) account, or use your existing Google account.

Expected Result: You are signed into Google in your web browser.

### Step 2: Open Firebase Console

Go to https://console.firebase.google.com/ in your browser.

Sign in with your Google account if prompted.

Expected Result: You see the Firebase Console welcome page with a "Create a project" (or "Add project") button.

### Step 3: Create a New Firebase Project

Click "Create a project" (or "Add project").

Enter a project name, for example: smart-chicken-feeder. Click Continue.

Toggle Enable Google Analytics for this project to Off (Analytics is not needed for the feeder, and turning it off makes setup faster).

Click Create project, wait ~30 seconds for provisioning, and click Continue.

Expected Result: You land on your new project's Project Overview dashboard in Firebase Console.

### Step 4: Register the Android Application (and Web App)

On the Project Overview page, click the circular Android icon (+ Add app → Android) near the top.

In Android package name, enter the exact package ID used by this repository (app/android/app/build.gradle.kts):

com.smartfeeder.smart_chicken_feeder

In App nickname (optional), type Smart Chicken Feeder Android.

Click Register app.

Download google-services.json if you plan to rebuild Android native resources (store it locally; never commit secrets publicly), then click Next → Next → Continue to console.

(Optional for Web / iPhone PWA) Click `+ Add app` → Web (`</>`), name it Smart Chicken Feeder Web, leave Firebase Hosting unchecked for now, and click Register app.

Expected Result: Your Project Overview now lists your registered Android (and Web) app(s).

### Step 5: Obtain Your Firebase Configuration Values

Click the Gear icon (⚙️) beside Project Overview in the top-left sidebar and click Project settings.

Under the General tab, scroll down to Your apps and record these values into a temporary private notepad on your PC:

Project ID (e.g., smart-chicken-feeder-xxxxx)

Web API Key (AIza... — shown at the top under Web API Key; if blank, click Authentication first to auto-generate it)

App ID, Messaging Sender ID, and Storage Bucket

Expected Result: You have your Project ID and Web API Key ready to paste into your local configuration files in Steps 9 and 10.

### Step 6: Enable Firebase Realtime Database (`asia-southeast1`)

In the left sidebar, click Build → Realtime Database.

Click Create Database.

Under Realtime Database location, choose Singapore (`asia-southeast1`) (closest low-latency region for Southeast Asia; matches the repository architecture) and click Next.

Select Start in locked mode (we will paste the exact authenticated rules in Step 7) and click Enable.

Copy the Database URL shown at the top of the data viewer (for example: https://<YOUR_PROJECT_ID>-default-rtdb.asia-southeast1.firebasedatabase.app).

Expected Result: You see an empty Realtime Database tree displaying null under your database URL.

### Step 7: Configure Database Security Rules (Without Leaving Database Public!)

Inside Realtime Database, click the Rules tab at the top.

Replace the entire contents of the editor with the repository's verified `database.rules.json` file:

{ "rules": { ".read": false, ".write": false, "telemetry": { ".read": "auth != null", ".write": "auth != null", ".validate": "newData.hasChildren(['feedLevelPercent', 'batteryVolts', 'status'])" }, "commands": { ".read": "auth != null", ".write": "auth != null", "dispense": { ".validate": "!newData.exists() ¦¦ (newData.hasChildren(['action']) && newData.child('action').isString())" } }, "schedules": { ".read": "auth != null", ".write": "auth != null" }, "logs": { ".read": "auth != null", ".write": "auth != null", ".indexOn": ["timestamp", "triggerType", "isSuccess"], "$logId": { ".validate": "newData.hasChildren(['timestamp', 'targetGrams', 'actualGrams', 'triggerType'])" } }, "feeders": { "$feederId": { ".read": "auth != null", ".write": "auth != null" } } } }

Click Publish.

Why These Rules Are Safe: Root .read and .write are false, blocking anonymous unauthenticated bots on the public internet from wiping or tampering with your database. Only clients holding a valid Firebase Authentication token (auth != null) can read/write feeder nodes, and .validate rules block malformed payloads.

Expected Result: Firebase displays "Rules published" with zero syntax errors.

### Step 8: Enable Firebase Authentication (Anonymous + Email/Password)

Both the Flutter mobile app (FirebaseAuth.instance.signInAnonymously()) and the ESP32 (ensureFirebaseIdToken() in firmware/src/main.cpp) authenticate with Firebase Auth before reading or writing database paths!

In the left sidebar, click Build → Authentication → Get started.

On the Sign-in method tab, click Anonymous, toggle Enable to On, and click Save.

(Recommended for dedicated ESP32 device credentials) Click Add new provider → Email/Password, toggle Enable to On, and click Save. Then switch to the Users tab → click Add user → create a device account such as esp32-feeder@yourdomain.com with a strong password. (If you leave Email/Password blank in `secrets.h`, the ESP32 automatically falls back to Anonymous Sign-Up.)

Expected Result: Under Authentication → Sign-in method, Anonymous (and optionally Email/Password) show status Enabled.

### Step 9: Configure ESP32 Firebase Credentials *(Developer Step)*

On your computer inside the repository folder, copy the template file firmware/include/secrets.example.h to create firmware/include/secrets.h.

Open firmware/include/secrets.h in Notepad or VS Code and fill in your values (note that FIREBASE_DATABASE_URL has no https:// and no trailing slash, matching secrets.example.h):

#pragma once // 2.4 GHz Farm Wi-Fi Credentials #define WIFI_SSID "<YOUR_2.4GHZ_WIFI_NAME>" #define WIFI_PASSWORD "<YOUR_WIFI_PASSWORD>" // Firebase Realtime Database Host (WITHOUT https:// or trailing /) #define FIREBASE_DATABASE_URL "<YOUR_PROJECT_ID>-default-rtdb.asia-southeast1.firebasedatabase.app" #define FIREBASE_API_KEY "<YOUR_FIREBASE_WEB_API_KEY>" // Optional: Device Email/Password created in Step 8 (leave "" to use Anonymous Auth) #define FIREBASE_USER_EMAIL "" #define FIREBASE_USER_PASSWORD ""

Save firmware/include/secrets.h.

Expected Result: firmware/include/secrets.h exists locally and is automatically ignored by Git so your keys stay private.

### Step 10: Configure Flutter Firebase Integration *(Developer Step)*

If you are compiling the Flutter app from source for your own Firebase project:

Create or edit app/lib/firebase_options.dart (git-ignored) using the values from Step 5 and Step 6:

import 'package:firebase_core/firebase_core.dart' show FirebaseOptions; import 'package:flutter/foundation.dart' show defaultTargetPlatform, kIsWeb, TargetPlatform; class DefaultFirebaseOptions { static FirebaseOptions get currentPlatform { if (kIsWeb) return web; switch (defaultTargetPlatform) { case TargetPlatform.android: return android; case TargetPlatform.iOS: return ios; default: return web; } } static const FirebaseOptions web = FirebaseOptions( apiKey: '<YOUR_FIREBASE_WEB_API_KEY>', appId: '<YOUR_WEB_APP_ID>', messagingSenderId: '<YOUR_SENDER_ID>', projectId: '<YOUR_PROJECT_ID>', authDomain: '<YOUR_PROJECT_ID>.firebaseapp.com', databaseURL: 'https://<YOUR_PROJECT_ID>-default-rtdb.asia-southeast1.firebasedatabase.app', storageBucket: '<YOUR_PROJECT_ID>.firebasestorage.app', ); static const FirebaseOptions android = FirebaseOptions( apiKey: '<YOUR_FIREBASE_ANDROID_API_KEY>', appId: '<YOUR_ANDROID_APP_ID>', messagingSenderId: '<YOUR_SENDER_ID>', projectId: '<YOUR_PROJECT_ID>', databaseURL: 'https://<YOUR_PROJECT_ID>-default-rtdb.asia-southeast1.firebasedatabase.app', storageBucket: '<YOUR_PROJECT_ID>.firebasestorage.app', ); static const FirebaseOptions ios = FirebaseOptions( apiKey: '<YOUR_FIREBASE_IOS_API_KEY>', appId: '<YOUR_IOS_APP_ID>', messagingSenderId: '<YOUR_SENDER_ID>', projectId: '<YOUR_PROJECT_ID>', databaseURL: 'https://<YOUR_PROJECT_ID>-default-rtdb.asia-southeast1.firebasedatabase.app', storageBucket: '<YOUR_PROJECT_ID>.firebasestorage.app', iosBundleId: 'com.smartfeeder.smartChickenFeeder', ); }

(Alternatively, run `flutterfire configure` via the Firebase CLI to generate `app/lib/firebase_options.dart` automatically.)

Expected Result: Running cd app; dart analyze lib/ reports No issues found!

### Step 11: Seed or Inspect Initial Database Structure (Optional)

You do not need to manually type data into Firebase—as soon as the ESP32 boots and connects to Wi-Fi, it automatically creates /telemetry! If you wish to inspect the clean schema template, see `data.json` in the repository root.

### Step 12: Test Connectivity and Verify App ↔ ESP32 Synchronization

Power on the flashed ESP32 (see Chapter 6).

Open Firebase Console → Realtime Database → Data. Within 10 seconds, you should see /telemetry light up green and update lastHeartbeat.

Open the Smart Chicken Feeder mobile app on your phone. Confirm the top-right badge turns green and says `Online`.

In the app's Schedules tab, add a test schedule. Watch /schedules appear immediately inside Firebase Console, and within 30 seconds observe [Cloud] Synced 1 schedule(s) from Firebase -> NVS in the ESP32 Serial Monitor.

Expected Result: Live data flows both ways between the phone, Firebase Realtime Database, and the ESP32!

## 4.3 Actual Database Schema and Example JSON Payloads

Below is an exact snapshot of the production Firebase Realtime Database tree generated by firmware/src/main.cpp and app/lib/services/hardware/firebase_hardware_adapter.dart:

{ "telemetry": { "feedLevelPercent": 84.0, "batteryVolts": 12.68, "batteryPercent": 92, "currentWeightGrams": 0.0, "status": "idle", "isSolarCharging": true, "lastHeartbeat": 1761840000000, "lastFedTimestamp": 1761820200000, "lastFedGrams": 100.4 }, "commands": { "dispense": { "id": "cmd_1761840050123", "action": "DISPENSE", "durationSeconds": 5, "targetGrams": 100.0, "triggerType": "Manual", "timestamp": 1761840050123, "status": "pending" }, "latest": { "id": "cmd_1761840050123", "action": "DISPENSE", "durationSeconds": 5, "targetGrams": 100.0, "actualGrams": 100.8, "status": "completed", "updatedAt": 1761840056000 } }, "schedules": { "sch_1761800000000": { "label": "Morning Feeding", "hour": 7, "minute": 0, "durationSeconds": 5, "targetGrams": 100.0, "isEnabled": true, "daysOfWeek": [1, 2, 3, 4, 5, 6, 7] } }, "logs": { "-Of9xSamplePushKey": { "timestamp": 1761820200000, "targetGrams": 100.0, "actualGrams": 100.4, "durationSeconds": 5, "triggerType": "Schedule (Morning Feeding)", "isSuccess": true, "batteryVolts": 12.65, "feedLevelPercent": 84.0 } } }

## 4.4 Common Firebase Errors and Solutions

| Firebase Error / Symptom | Root Cause | Exact Fix |
| --- | --- | --- |
| `HTTP 401 Unauthorized` in ESP32 Serial Monitor or "permission denied" banner in App | Anonymous (or Email/Password) sign-in provider is disabled in Firebase Console, or FIREBASE_API_KEY is wrong. | Enable Anonymous in Firebase Console → Authentication → Sign-in method and verify FIREBASE_API_KEY in secrets.h. |
| `HTTP 400 Bad Request` when writing /telemetry or /logs | Payload missing a required field enforced by .validate in database.rules.json. | Ensure database.rules.json matches Step 7 and firmware is unmodified. |
| ESP32 prints `Firebase Auth failed: -1` | FIREBASE_DATABASE_URL includes https:// or ESP32 Wi-Fi has no internet access (captive portal / DNS failure). | Remove https:// and any trailing / from FIREBASE_DATABASE_URL in secrets.h; verify router has internet. |
| App connects to Firebase, but says `Feeder controller offline` | Firebase Cloud works, but /telemetry/lastHeartbeat is null or >= 3 minutes old because the ESP32 is off or disconnected from Wi-Fi. | Power on the ESP32 and verify [WiFi] Connected! + [Firebase] Authenticated in Serial Monitor. |

# Chapter 5: Frontend and Backend Communication

This chapter traces all 10 core communication operations across the Flutter frontend (app/lib/), Firebase Realtime Database, and ESP32 firmware (firmware/src/main.cpp), referencing exact classes and source files without modifying any implementation code.

## 5.1 Operation 1: Flutter Application Initialization

Trigger: User taps the Smart Chicken Feeder icon on Android or opens the Web App URL.

Handling Component: main() and _SmartFeederAppBootstrapState._initializeServices() in `app/lib/main.dart`.

Database Path / Storage: Local device SharedPreferences (smart_feeder_settings.json backup on IO platforms).

Transmitted Data: Loads theme_mode, feeder_name, low_feed_threshold, low_battery_threshold, hopper_capacity_kg, onboarding_complete, and hardware_transport. Initializes AlertNotificationService.

Expected Response: Returns populated SettingsService instance and requests Android POST_NOTIFICATIONS permission on supported mobile OS versions.

UI Display: Shows branded splash screen while initializing, then routes to OnboardingScreen (if first launch) or MainNavigationShell.

## 5.2 Operation 2: Firebase Core & Authentication Initialization

Trigger: Immediately during _initializeServices() in app/lib/main.dart and FirebaseHardwareAdapter.initialize() in `app/lib/services/hardware/firebase_hardware_adapter.dart`.

Handling Component: Firebase.initializeApp(options: DefaultFirebaseOptions.currentPlatform) followed by FirebaseAuth.instance.signInAnonymously() (with HTTPS REST fallback to identitytoolkit.googleapis.com/v1/accounts:signUp?key=...).

Database Path / Endpoint: Firebase Auth IdentityToolkit service.

Transmitted Data: {"returnSecureToken": true} (REST fallback) or native SDK anonymous credential request.

Expected Response: Authenticated UserCredential / idToken satisfying "auth != null" in database.rules.json.

UI Display: Transitions FeederConnection from connecting to online once the first RTDB listener responds; if auth fails (e.g., disabled in console), ConnectionBanner displays the exact permission error with a Retry button.

## 5.3 Operation 3: Database Connection & Dual-Transport Strategy

Trigger: Startup adapter binding (FeederService) or user changing Hardware Transport dropdown in SettingsScreen (_switchHardwareTransport() in `app/lib/screens/settings_screen.dart`).

Handling Component: FeederService.switchAdapter() in `app/lib/services/feeder_service.dart` swapping between FirebaseHardwareAdapter and RestHardwareAdapter (`app/lib/services/hardware/rest_hardware_adapter.dart`).

Database Path / Endpoint:

Cloud: /info/connected, /telemetry, /schedules, /logs, /commands/latest.

SoftAP: GET http://192.168.4.1/api/telemetry (every 2s) and GET http://192.168.4.1/api/logs (every 6s).

Expected Response: Continuous Stream<FeederTelemetry>, Stream<List<FeedingSchedule>>, Stream<List<FeedingLog>>, and Stream<FeederConnection>.

UI Display: Displays confirmation snackbar ("Hardware transport switched to ..."), updates header status pill, and refreshes dashboard cards.

## 5.4 Operation 4: Reading Live Sensor Telemetry

Trigger: Automatic 10s ESP32 heartbeat (pushTelemetryToFirebase() in firmware/src/main.cpp lines 1054–1086), 1.5s mid-dispense stream, or user tapping Sync Feeder Telemetry (`↻`) / pulling down on DashboardScreen.

Handling Component: ESP32 buildTelemetryJson() → Firebase /telemetry → Flutter FeederTelemetry.fromMap() (`app/lib/models/telemetry.dart`).

Database Path / Endpoint: PUT /telemetry.json (Cloud) or GET http://192.168.4.1/api/telemetry (SoftAP).

Transmitted Data:

{ "feedLevelPercent": 85.0, "batteryVolts": 12.65, "batteryPercent": 90, "currentWeightGrams": 0.0, "status": "idle", "isSolarCharging": true, "lastHeartbeat": 1761840000000, "lastFedTimestamp": 1761820200000, "lastFedGrams": 100.4 }

Expected Response: HTTP 200 OK on ESP32; real-time stream emission in FeederService.

UI Display: Updates Hopper Level progress bar, ~kg remaining, ~days left, Solar System (Charging/Idle), Battery % & Volts, and snackbar on manual refresh.

## 5.5 Operation 5: Sending Manual Feeding & Emergency Stop Commands

Trigger: User taps `DISPENSE <X>s NOW` → `Confirm & Dispense` (or `CANCEL / EMERGENCY STOP`) on DashboardScreen.

Handling Component: FeederService.triggerManualFeed() / cancelDispensing() → sendCommand() in active adapter → pollFirebaseCommands() & processIncomingCommandJson() (firmware/src/main.cpp lines 919–1050) or POST /api/dispense / /api/stop.

Database Path / Endpoint:

Cloud Dispense: Writes /commands/dispense and /commands/latest.

Cloud Stop: Writes /commands/stop and updates /commands/dispense.

SoftAP: POST http://192.168.4.1/api/dispense or POST http://192.168.4.1/api/stop.

Transmitted Data:

{ "id": "cmd_1761840050123", "action": "DISPENSE", "durationSeconds": 5, "targetGrams": 0.0, "triggerType": "Manual", "timestamp": 1761840050123, "status": "pending" }

(Note: When `targetGrams` is `0.0` from the duration slider, ESP32 runs `dispenseFeedDuration(durationSeconds)` while measuring actual grams on the HX711 if connected; when `targetGrams > 0`, ESP32 runs closed-loop `dispenseFeed(targetGrams)`.)

Expected Response: ESP32 immediately `DELETE`s `/commands/dispense.json` before actuating the servo, patches /commands/latest to "inProgress", runs the gate, and patches /commands/latest to "completed", "aborted", or "failed" with actualGrams.

UI Display: Shows green dispatch snackbar, animated blue Dispensing in Progress... progress banner with live weight readout, and red CANCEL / EMERGENCY STOP button until hardware completion.

## 5.6 Operation 6: Configuring Feeding Schedules

Trigger: User adds, edits, toggles, deletes, or undoes deletion of a routine on SchedulesScreen (`app/lib/screens/schedules_screen.dart`).

Handling Component: FeederService.addSchedule() / updateSchedule() / toggleSchedule() / deleteSchedule() → syncSchedules() → ESP32 syncSchedulesFromFirebase() (every 30s) or POST /api/schedules.

Database Path / Endpoint: /schedules (keyed by schedule id, e.g., /schedules/sch_1761800000000) or POST http://192.168.4.1/api/schedules.

Transmitted Data: Map of FeedingSchedule objects (label, hour, minute, durationSeconds, targetGrams, isEnabled, daysOfWeek).

Expected Response: Persisted in Firebase /schedules and saved into ESP32 NVS (prefs.putString("schedules", ...)). When all schedules are deleted, /schedules is removed cleanly and ESP32 clears NVS (scheduleCount = 0).

UI Display: Updates active daily routine counter, total daily runtime/grams banner, and Dashboard "Next Automated Feeding" countdown.

## 5.7 Operation 7: Receiving Machine Status & Online Heartbeat

Trigger: Every telemetry update + periodic freshness check in FeederService.isDeviceOnline and FeederTelemetry.isOnline (`app/lib/models/telemetry.dart` lines 55–59).

Handling Component: FeederTelemetry.isOnline checks lastHeartbeat != null and DateTime.now().difference(lastHeartbeat!).inMinutes < 3.

Database Path: /telemetry/status, /telemetry/sensorError, and /telemetry/lastHeartbeat.

Expected Response: Maps status string to FeederStatus enum (idle, dispensing, lowFeed, lowBattery, offline, error).

UI Display: Updates top-right status pill (Online green, Dispensing amber, Connecting blue, Offline red) and slides down ConnectionBanner if heartbeat goes stale (>= 3 minutes).

## 5.8 Operation 8: Updating Feeding History & CSV Export

Trigger: ESP32 finishes any manual or scheduled dispense (emitFeedingLog() in firmware/src/main.cpp lines 473–509), or user taps Download Icon (`⬇`) on HistoryScreen.

Handling Component: ESP32 POST /logs.json → Flutter FeedingLog.fromMap() (`app/lib/models/feeding_log.dart`) → FileExporter.downloadTextFile() (`app/lib/services/file_export/file_exporter_io.dart`).

Database Path / Endpoint: /logs/{pushId} (Cloud) or GET http://192.168.4.1/api/logs (25-entry ring buffer in SoftAP mode).

Transmitted Data: timestamp, targetGrams, actualGrams, durationSeconds, triggerType, isSuccess, batteryVolts, feedLevelPercent.

Expected Response: Sorted newest-first in FeederService.history; CSV exporter prepends UTF-8 BOM (\uFEFF) and writes feeding_logs_YYYYMMDD_HHmmss.csv to Downloads.

UI Display: Recalculates Total Feeds, Avg Accuracy (%), and Total Dispensed in the Chapter IV Data summary card, renders variance badges (<= 3.0g green), and confirms CSV file path via snackbar.

## 5.9 Operation 9: Handling Real-Time Changes & Local OS Alerts

Trigger: Live telemetry or new feeding log arrives while telemetry.isOnline == true.

Handling Component: AlertNotificationService.evaluateTelemetry() & evaluateFeedingLog() (`app/lib/services/alert_notification_service.dart`).

Database Path: Evaluates /telemetry (status, sensorError, feedLevelPercent, batteryPercent, batteryVolts) and /logs (isSuccess).

Expected Response: Emits FeederAlert on alertStream and invokes FlutterLocalNotificationsPlugin.show() on channel feeder_safety_alerts (throttled by 15-minute cooldown and +2% recovery hysteresis).

UI Display: High-priority Android/iOS system tray alert + colored warning banner at the top of DashboardScreen.

## 5.10 Operation 10: Handling Communication Failures & Offline Caching

Trigger: Phone loses internet, router power drops, or Firebase Auth idToken expires after 1 hour.

Handling Component:

Flutter: FeederService._persistStateToLocalCache() & _hydrateFromLocalCache() cache last known telemetry, schedules, and up to 100 logs in SharedPreferences.

ESP32: ensureFirebaseIdToken() proactively refreshes tokens at 45 minutes (2,700,000 ms) and immediately retries once if any Firebase HTTP call returns 401. Every 15s, loop() runs non-blocking WiFi.reconnect().

UI Display: App never crashes or shows a blank white screen—it displays cached schedules/logs immediately alongside ConnectionBanner ("Showing last known data from ... Saved schedules keep running on the feeder") with a one-tap Retry button.

# Chapter 6: ESP32 and Machine Configuration

Who Needs This Chapter? Follow this chapter if you are assembling the electronics box or uploading the firmware program onto a brand-new ESP32 board (Developer / Technician Procedure, written step-by-step so a beginner with zero coding experience can follow every click).

## 6.1 What Is the ESP32? (Plain-English Introduction)

The ESP32-WROOM-32 DevKit V1 is a pocket-sized computer board inside your chicken feeder. Unlike a normal computer, it does not have a screen or keyboard—instead, you plug it into a Windows PC or laptop one time using a USB cable to copy the feeder's control program ("firmware") onto it. Once programmed, the ESP32 runs automatically whenever 5.0V solar battery power is switched on.

## 6.2 Required Hardware Components, Cables, and Tools

### Electronic & Solar Components

1x ESP32-WROOM-32 Development Board (30-pin DevKit V1, esp32dev compatible).

1x MG996R Metal-Gear High-Torque Servo Motor (180° positional version, not 360° continuous rotation).

1x 5 kg Straight Bar Load Cell + HX711 Amplifier Module.

1x HC-SR04 Ultrasonic Distance Sensor.

1x DS3231 I2C Real-Time Clock Module + 1x CR2032 3V Coin Battery.

1x LM2596 DC-DC Step-Down Buck Converter Module (with small brass adjustment screw).

1x 12V Solar Panel (20W–50W recommended) + 1x 12V PWM Solar Charge Controller + 1x 12V Rechargeable Battery (7Ah–12Ah SLA or LiFePO4).

Resistors & Capacitor:

For Battery Voltage Divider (GPIO 34): 1x `40 kΩ` resistor (or two 20 kΩ in series) and 1x `10 kΩ` resistor.

For Ultrasonic Echo Divider (GPIO 18): 1x `1 kΩ` resistor and 1x `2 kΩ` resistor (or two 1 kΩ in series).

For Servo Surge Filtering: 1x `1000 µF` (`16V` or `25V`) electrolytic capacitor across the 5.0V servo power rail and GND.

### Required Cables and Tools

1x Data-Capable USB Cable (Micro-USB or USB-C depending on your ESP32 board—Important: cheap "charge-only" cables from small fans/lamps do not carry data and will not show a COM port!).

1x Digital Multimeter (mandatory to tune the LM2596 buck converter to 5.0V before connecting the ESP32).

1x Small Flathead Jeweler's Screwdriver (for turning the brass screw on the LM2596 and charge controller terminals).

Jumper Wires / Screw Terminal Block / Soldering Iron + Weatherproof Enclosure (IP65 junction box recommended for farm coops).

## 6.3 Critical Electrical & Power Safety Warnings

⚠️ WARNING 1 — TUNE THE LM2596 BUCK CONVERTER TO 5.0V BEFORE CONNECTING ANYTHING! Brand-new LM2596 modules ship from the factory set to output the full input voltage (12V–14V). Connecting 12V to the ESP32 5V/VIN pin or the MG996R servo will instantly burn out and destroy both components. Connect only the 12V battery to LM2596 IN+ / IN-, place your multimeter probes on OUT+ / OUT-, and turn the tiny brass screw counter-clockwise (sometimes 15–20 turns) until the meter reads `5.00V DC` (`±0.05V`) before wiring the ESP32 or servo!

⚠️ WARNING 2 — NEVER POWER THE MG996R SERVO FROM AN ESP32 PIN! The MG996R motor pulls up to `2.5 Amps` when starting or pushing against pellets. Wire the servo's Red (+) and Brown/Black (GND) power wires directly to the `LM2596` `5.0V` output terminals, with a 1000 µF capacitor across +5V and GND. Only the Orange/Yellow signal wire connects to ESP32 `GPIO 13`.

⚠️ WARNING 3 — COMMON GROUND (`GND`) IS MANDATORY! Every ground wire in the system (12V Battery -, LM2596 OUT-, ESP32 GND, Servo GND, HX711 GND, HC-SR04 GND, DS3231 GND, and resistor divider bottom legs) must tie together to one shared Ground bus. Missing common ground causes erratic servo twitching and false sensor numbers.

⚠️ WARNING 4 — PROTECT `GPIO 18` FROM 5V ULTRASONIC ECHO! ESP32 pins operate at 3.3V logic and are not 5V-tolerant. Connect HC-SR04 ECHO → 1 kΩ resistor → `GPIO 18`, and also connect `GPIO 18` → 2 kΩ resistor → `GND`. This safely drops the 5V echo signal down to 3.3V.

## 6.4 Step-by-Step Firmware Setup, Configuration & Upload (Steps 1–14)

The repository's firmware (firmware/platformio.ini) uses Visual Studio Code + PlatformIO, which is much easier for beginners than Arduino IDE because PlatformIO automatically downloads the exact ESP32 board support and all 5 required libraries for you without manual .zip hunting!

### Step 1: Install Visual Studio Code (VS Code)

On a Windows PC with internet access, open https://code.visualstudio.com/.

Click Download for Windows, run the downloaded installer, and click Next → Install → Finish.

Expected Result: Visual Studio Code opens to its Welcome tab.

### Step 2: Install the PlatformIO IDE Extension

Inside VS Code, click the Extensions icon on the left sidebar (four little squares, or press Ctrl+Shift+X).

In the search box at the top, type `PlatformIO IDE`.

Click the official PlatformIO IDE result (orange alien ant icon) and click the blue Install button.

Wait 2–4 minutes while PlatformIO installs its core tools in the bottom-right corner. When prompted, click Reload Now (or close and reopen VS Code).

Expected Result: An alien icon (PlatformIO) appears on the left sidebar of VS Code, and a small Checkmark (`✓`) and Right-Arrow (`→`) appear in the blue/bottom status bar.

### Step 3: Install USB-to-Serial Driver (If Windows Does Not Recognize Your ESP32)

Most ESP32 boards use a small USB chip (CP2102 or CH340).

Plug your ESP32 into your PC's USB port.

Right-click the Windows Start button → click Device Manager → expand Ports (COM & LPT).

If you see Silicon Labs CP210x USB to UART Bridge (COM3) or USB-SERIAL CH340 (COM4), your driver is already installed—note that COM number!

If instead you see a yellow warning triangle under Other devices, download and install the Silicon Labs CP210x Universal Windows Driver (or WCH CH341SER driver), unplug the ESP32, and plug it back in.

Expected Result: Windows Device Manager shows a valid COM port (for example, COM3, COM4, or COM5) when the ESP32 is plugged in.

### Step 4: Download or Open the Project Folder

Download the repository from https://github.com/Jehooooo/SMART-MOBILE-BASED-SOLAR-POWERED-CHICKEN-FEEDING-SYSTEM (click the green `<> Code` button → Download ZIP → extract the ZIP to Documents\AutomaticDispensing).

In VS Code, click File → Open Folder..., select the `firmware` subfolder (Documents\AutomaticDispensing\firmware), and click Select Folder. (Important: Open the `firmware` folder directly so PlatformIO immediately detects `platformio.ini`!)

Expected Result: The left Explorer panel shows include, src, and platformio.ini. PlatformIO automatically downloads the espressif32 board package and all required libraries listed in platformio.ini:

madhephaestus/ESP32Servo@^3.0.5

bogde/HX711@^0.7.5

adafruit/RTClib@^2.1.4

mobizt/Firebase Arduino Client Library for ESP8266 and ESP32@^4.4.14

bblanchon/ArduinoJson@^7.2.0

### Step 5: Verify Board Selection (`esp32dev`)

Click platformio.ini in the left panel to view it.

Confirm lines 12–15 already configure the exact board and serial speed:

[env:esp32dev] platform = espressif32 board = esp32dev framework = arduino monitor_speed = 115200

You do not need to change anything in platformio.ini.

Expected Result: PlatformIO bottom bar shows Default (firmware) / env:esp32dev.

### Step 6: Create Your Private `secrets.h` File

In the left Explorer panel under include/, right-click `secrets.example.h` and click Copy.

Right-click the `include` folder and click Paste.

Right-click the copied file (secrets.example copy.h), click Rename, and name it exactly:

secrets.h

Expected Result: You now see include/secrets.h in the file list (greyed out slightly because Git protects it from being uploaded publicly).

### Step 7: Enter Your Farm Wi-Fi and Firebase Credentials

Click include/secrets.h to open it in the editor.

Type your 2.4 GHz Wi-Fi name (WIFI_SSID) and Wi-Fi password (WIFI_PASSWORD) inside the quotation marks.

Paste your FIREBASE_DATABASE_URL (without https://) and FIREBASE_API_KEY from Chapter 4:

#pragma once #define WIFI_SSID "<YOUR_2.4GHZ_WIFI_SSID>" #define WIFI_PASSWORD "<YOUR_WIFI_PASSWORD>" #define FIREBASE_DATABASE_URL "<YOUR_PROJECT_ID>-default-rtdb.asia-southeast1.firebasedatabase.app" #define FIREBASE_API_KEY "<YOUR_FIREBASE_WEB_API_KEY>" #define FIREBASE_USER_EMAIL "" #define FIREBASE_USER_PASSWORD ""

(Note: If your farm has zero internet and you only plan to use off-grid Direct Wi-Fi mode, you can leave `WIFI_SSID` as `""`—the firmware will compile cleanly and run `SmartFeeder-AP`!)

Press `Ctrl+S` to save secrets.h.

Expected Result: secrets.h has no unsaved dot next to its tab name.

### Step 8: Review GPIO Pins and Physical Dimensions (`include/config.h`)

Open include/config.h.

Verify that your physical hopper height matches HOPPER_FULL_CM (5.0f cm from lid sensor when full) and HOPPER_EMPTY_CM (30.0f cm from lid sensor to bottom floor when empty). If your container is taller (for example, 45 cm deep when empty), change 30.0f to 45.0f and press Ctrl+S.

Keep all GPIO pin numbers (13, 16, 17, 5, 18, 21, 22, 34, 2) unchanged so they match your wiring diagram.

Expected Result: config.h matches your physical container geometry.

### Step 9: Test-Compile the Firmware (Checkmark Button)

Click the Checkmark (`✓`PlatformIO: Build) icon in the bottom-left blue status bar of VS Code.

Watch the terminal panel at the bottom compile the C++ code.

Expected Result: After 15–45 seconds, the terminal prints green `[SUCCESS] Took X.XX seconds` (using ~15% RAM and ~53% Flash). If it says secrets.h: No such file, repeat Step 6.

### Step 10: Connect ESP32 via USB (Disconnect External 5V Power First, or Share Safe Power)

Safety Tip: For your very first bench upload, you can plug the bare ESP32 into your PC via USB even before wiring the external battery, OR with external power off.

Plug the USB data cable into your PC and into the ESP32's USB port.

PlatformIO automatically detects your COM port. (To pick a specific port manually, click the Plug icon on the bottom status bar and choose your ESP32 COM port from Step 3.)

Expected Result: The red power LED on the ESP32 board lights up steadily.

### Step 11: Upload (Flash) the Firmware to the ESP32

Click the Right-Arrow (`→` PlatformIO: Upload) icon in the bottom-left status bar.

Watch the terminal compile and print Connecting.........

Beginner Tip: On some ESP32 DevKit V1 boards, if Connecting........_____ repeats for more than 5 seconds, press and hold the small `BOOT` button on the ESP32 board for 2 seconds until Writing at 0x00010000... (10 %) starts, then let go!

Expected Result: Terminal reaches Writing at ... (100 %), prints Hard resetting via RTS pin..., and displays green `[SUCCESS]`.

### Step 12: Open the Serial Monitor (`115200` Baud)

Click the Plug icon (`🔌` PlatformIO: Serial Monitor) in the bottom status bar.

Press the small `EN` (or `RST`) button on the ESP32 board once to watch a fresh startup from line 1.

Expected Result: You see readable English text (not garbled symbols, confirming 115200 baud) starting with:

==================================================== SMART SOLAR CHICKEN FEEDER - FIRMWARE v1.0.0 ==================================================== [NVS] Calibration factor: 420.00 ¦ Saved schedules: 0 [WiFi] SoftAP ready: SSID=SmartFeeder-AP IP=192.168.4.1 [HTTP] Local REST server & web portal started on http://192.168.4.1

### Step 13: Confirm Wi-Fi and Firebase Connection in Serial Monitor

Look at the lines right below [HTTP] Local REST server... in the Serial Monitor:

If WIFI_SSID is in range and correct, you will see:

[WiFi] Connecting to station SSID: <YOUR_WIFI> ..... [WiFi] Connected! Station IP: 192.168.1.XX [Time] NTP clock synchronized: 2026-10-31 08:00:00 [Firebase] Signing in anonymously... [Firebase] Authenticated (token length: 8xx) [Safety] Boot command queue purged (/commands/dispense & /commands/stop).

Every 10 seconds, you will also see live sensor readings printed:

[Telemetry] Feed: 85.0% ¦ Bat: 12.65V (90%) ¦ Weight: 0.0g ¦ Status: idle ¦ Err: none

## 6.5 Testing Individual Sensors, the Servo Motor, and Firebase Sync

Before bolting the electronics into the chicken coop, verify every part right from the Serial Monitor or Mobile Diagnostics Suite:

Test the Weighing Scale (`HX711`):

Click inside the Serial Monitor window and press `t` on your keyboard (no Enter needed, or press Enter).

Expected Result: Serial prints [Serial] Scale tared to 0.0g. Place a 500g water bottle on the load cell; wait 10s and confirm Weight: increases in the [Telemetry] readout. (If it prints Err: hx711_disconnected, check GPIO 16 / GPIO 17 and HX711 VCC/GND.)

Test the Ultrasonic Hopper Sensor (`HC-SR04`):

Hold a flat book 5 cm in front of the HC-SR04 "eyes" → Feed: should read `100.0%`.

Move the book 30 cm away → Feed: should drop to `0.0%`.

Test the Servo Gate (`MG996R`) & Emergency Stop *(With External 5.0V Buck Power Connected!)*:

Press `f` in Serial Monitor to trigger a test 200g dispense → Servo rotates smoothly to `90°`.

Immediately press `s` in Serial Monitor to test Emergency Stop → Servo snaps back to `0°` and prints [Safety] Emergency stop requested via Serial.

Test Firebase Synchronization:

Watch Firebase Console → Realtime Database → Data while pressing t or moving your hand in front of the ultrasonic sensor. Confirm /telemetry updates live in your browser within 10 seconds.

# Chapter 7: Physical Machine and Mobile Application Integration

## 7.1 Hardware Assembly

Follow this physical assembly order so mechanical parts and sensors do not interfere with each other:

Step 1: Mount the Hopper and Gravity Gate

Secure the feed hopper (3 kg – 15 kg container) vertically onto your feeder frame above the chicken trough.

Attach the MG996R servo horn to the sliding/pivoting chute gate at the bottom of the hopper.

Before tightening the horn screw, power on the programmed ESP32 (which commands `0°` at boot) and align the gate flap so `0°` completely seals the chute and `90°` opens it cleanly without hitting the frame.

Expected Result: The feed gate rests flush and sealed in its 0° position with zero mechanical binding.

Step 2: Mount the Load Cell Weighing Tray (`HX711`)

Bolt one end of the 5 kg aluminum load cell bar to the rigid lower frame (observe the arrow sticker on the load cell pointing downward in the direction of gravity!).

Bolt the feed catch/weighing tray to the opposite floating end of the bar using spacer washers so the tray hangs freely beneath the chute outlet.

Verify nothing rubs against the sides of the weighing tray. Wire the 4 load-cell wires into the HX711 (Red → E+, Black → E-, White → A-, Green → A+), and wire HX711 DT → GPIO 16, SCK → GPIO 17.

Expected Result: Pressing gently on the empty tray flexes only the load cell bar without scraping any walls.

Step 3: Mount the Ultrasonic Sensor (`HC-SR04`) Inside the Lid

Cut two circular openings (or mount a bracket) on the underside of the top hopper lid, aiming straight down toward the center of the feed pellets.

Keep the sensor at least 5 cm above the maximum pellet fill line (HOPPER_FULL_CM = 5.0) so pellets never touch the metal transducers.

Wire VCC → 5.0V, GND → Common GND, TRIG → GPIO 5, and ECHO through the 1 kΩ / 2 kΩ voltage divider to GPIO 18.

Expected Result: Closing the lid points the HC-SR04 perpendicular to the pellet surface.

Step 4: Install the DS3231 RTC and Solar Power Box

Snap a fresh CR2032 coin battery into the DS3231 module and wire VCC → 3.3V, GND → GND, SDA → GPIO 21, SCL → GPIO 22.

Mount the 12V solar panel on the coop roof angled toward the sun (10°–15° tilt, unshaded).

Inside a dry, ventilated weatherproof enclosure:

Wire Battery `+ / -` to the Solar Charge Controller Battery terminals FIRST (most charge controllers auto-detect 12V when the battery is connected first), then wire the Solar Panel `+ / -` to the Solar terminals.

Wire the 12V bus through an inline fuse/switch to LM2596 IN+ / IN- (pre-tuned to `5.00V`!) and to the 40 kΩ / 10 kΩ battery voltage divider feeding `GPIO 34`.

Wire LM2596 OUT+ (5.0V) and OUT- (Common GND) to the MG996R servo power rail (with 1000 µF capacitor) and ESP32 5V/VIN & GND.

Expected Result: Solar charge controller shows battery voltage (~12.4V–13.8V) and charging indicator; ESP32 boots cleanly with the gate closed at 0°.

## 7.2 Initial Machine Configuration

Before daily poultry use, perform three quick initial checks:

Clock Sync: Either let the ESP32 connect once to internet Wi-Fi (it automatically syncs UTC+8 NTP time into the DS3231 chip), OR connect via Direct Wi-Fi (SmartFeeder-AP) and sync time via /api/sync-clock.

Zero Tare: With the weighing tray completely empty, go to Settings → Zero / Tare Scale Sensor → `Tare (0.0g)`.

2-Point Scale Calibration: Open Settings → Hardware Diagnostics & Calibration Suite → `Open Suite`, place a 500g reference weight (e.g., an unopened 500 mL bottled water) on the tray, select `500g`, and tap `Save Factor`. The ESP32 permanently stores your exact load cell factor in NVS flash.

## 7.3 Wi-Fi Configuration & Actual Dual-Mode Behavior

Understanding how Wi-Fi actually works in firmware/src/main.cpp prevents 95% of setup confusion:

| Question | Actual Behavior Implemented in This Repository |
| --- | --- |
| Which Wi-Fi network does the ESP32 connect to? | A `2.4 GHz` Wi-Fi router or hotspot matching WIFI_SSID and WIFI_PASSWORD compiled in firmware/include/secrets.h. |
| Does the ESP32 automatically join my phone's hotspot? | Only if your phone's hotspot Name (SSID), Password, and 2.4 GHz band ("Maximize Compatibility" on iPhone / "2.4 GHz" on Android) exactly match WIFI_SSID and WIFI_PASSWORD in secrets.h. It does not guess random hotspots. |
| Must the phone and ESP32 be on the same Wi-Fi? | No in Cloud Mode (your phone can be on 4G/5G in another city while the ESP32 is on farm Wi-Fi). Yes in Direct Wi-Fi Mode (your phone must join the feeder's SmartFeeder-AP Wi-Fi). |
| Does the machine require internet access? | No! Scheduled feedings run 100% offline via the DS3231 clock, and manual control/calibration work 100% offline via SmartFeeder-AP (192.168.4.1). Internet is only required for remote Firebase Cloud monitoring outside Wi-Fi range. |
| Are Cloud Mode and Local Wi-Fi Mode automatic or manual in the app? | On the ESP32, both run simultaneously (WIFI_AP_STA). On the Mobile App, you select which mode to use via Settings → Hardware Transport (Firebase Realtime Cloud vs. Direct Wi-Fi HTTP). They are not silently swapped behind your back—you choose Cloud when remote, or Direct Wi-Fi when standing at an off-grid coop. |
| What happens when farm Wi-Fi disconnects? | The ESP32 keeps SmartFeeder-AP and DS3231 schedules running without interruption, and calls non-blocking WiFi.reconnect() every `15 seconds`. As soon as the router comes back online, the ESP32 automatically re-syncs NTP time, purges stale commands, and resumes 10s Firebase heartbeats. |

## 7.4 Mobile Application Connection

How the App Identifies the Machine:

In Firebase Cloud Mode, FirebaseHardwareAdapter subscribes to /telemetry, /schedules, /logs, and /commands/latest in the configured Firebase Realtime Database project.

In Direct Wi-Fi Mode, RestHardwareAdapter connects directly to http://192.168.4.1 once your phone joins Wi-Fi network `SmartFeeder-AP` (default firmware password in config.h: `chickens123`).

How the App Knows Whether the Machine Is Online:

Simply having internet on your phone is not enough to show Online! FeederService.isDeviceOnline checks two conditions simultaneously:

The transport link (FeederConnection) is online.

The hardware's lastHeartbeat timestamp inside /telemetry is less than `3 minutes` old (diff.inMinutes < 3).

If you unplug the ESP32, Firebase stays reachable, so within 3 minutes the app accurately switches to `Offline` and shows: "Feeder controller offline — Cloud database is reachable, but ESP32 has not sent a live heartbeat..."

How to Verify Both Sides Are Communicating:

Tap the circular Refresh (`↻`) button in the top-right bar of the Dashboard. You should see the snackbar: `Telemetry refreshed from hardware controller` and the green `Online` pill.

## 7.5 Complete Integration Test Checklist

Run through all 9 checks below before deploying the feeder in the poultry coop:

[ ] 1. ESP32 Wi-Fi Link: Serial Monitor shows [WiFi] Connected! Station IP: ... AND your phone sees SmartFeeder-AP in nearby Wi-Fi networks.

[ ] 2. ESP32 Firebase Link: Serial Monitor shows [Firebase] Authenticated with zero HTTP 401/400 errors.

[ ] 3. Flutter Firebase Link: Opening the Android app over mobile data (4G/5G) or Wi-Fi shows the green `Online` badge in the top-right corner of Dashboard.

[ ] 4. Live Sensor Readings: Placing your hand under the HC-SR04 sensor or pressing on the weighing tray updates Feed Hopper Level (%) and Live Weight Readout (g) in the app within 10 seconds (or immediately on tapping ↻).

[ ] 5. Manual Feeding & Emergency Stop: Selecting 3s • Quick (~60g) and tapping `DISPENSE 3s NOW` → `Confirm & Dispense` opens the servo gate to 90°, displays the blue progress bar, and closes cleanly back to 0°. Testing a 10s dispense and tapping `CANCEL / EMERGENCY STOP` mid-pour immediately shuts the gate to 0°.

[ ] 6. Scheduled Feeding (Online & Offline): Creating a schedule 2 minutes in the future in the Schedules tab syncs to the ESP32; unplugging/turning off the Wi-Fi router confirms the ESP32 still dispenses automatically at the exact minute via DS3231!

[ ] 7. Feeding History & CSV Export: Opening History & Logs displays the newly completed cycle with Target, Dispensed, Variance, Battery Volts, and Hopper %, and tapping Download (`⬇`) → `Download CSV` saves feeding_logs_*.csv to Downloads.

[ ] 8. Real-Time Safety Alerts: Moving the hopper empty (below 20%) triggers the yellow Low Feed Warning banner on Dashboard and a high-priority Android notification.

[ ] 9. Device Disconnection Detection: Powering off the ESP32 causes the app's status badge to flip from green `Online` to red `Offline` within 3 minutes and disables the `DISPENSE NOW` button so commands cannot pile up while offline.

# Chapter 8: Android Installation and User Guide

Audience: Ordinary Android smartphone users, farm owners, and coop staff. Zero programming knowledge or computer tools are required.

## 8.1 Required Android Version and Device Requirements

| Requirement | Minimum / Recommended Specification |
| --- | --- |
| Operating System | Android 6.0 (Marshmallow / API 21) or newer (Android 10 – Android 15 recommended). |
| Processor Architecture | 64-bit ARM (`arm64-v8a`) — used by 99% of modern phones (Samsung, Xiaomi, Redmi, POCO, Oppo, Vivo, Realme, Infinix, Tecno, Google Pixel). <br> 32-bit ARM (`armeabi-v7a`) — separate APK provided for older budget phones made before ~2018. |
| Free Storage Space | At least `75 MB` free internal storage (APK download is ~20.9 MB; installed app is ~52 MB). |
| Connectivity | Mobile Data (4G/5G) or Wi-Fi for Firebase Cloud Mode; 2.4 GHz Wi-Fi to join SmartFeeder-AP for Off-Grid Direct Mode. |
| Android Permissions | INTERNET, ACCESS_NETWORK_STATE (automatic), and Notifications (POST_NOTIFICATIONS on Android 13+ for low-feed/battery alerts). |

## 8.2 Step-by-Step APK Download and Installation (Steps 1–6)

### Step 1: Obtain the Official Release APK

On your Android phone (or on a PC to transfer later), open Google Chrome and visit the official GitHub Release page:

Release Page: https://github.com/Jehooooo/SMART-MOBILE-BASED-SOLAR-POWERED-CHICKEN-FEEDING-SYSTEM/releases/tag/v1.0.0

Tap `app-arm64-v8a-release.apk` (20.9 MB — Recommended for almost all phones).

(Only if your phone is very old and says "App not installed as package appears to be invalid/incompatible", download `app-armeabi-v7a-release.apk` [`18.5 MB`] instead.)

Expected Result: Chrome shows "Downloading file..." followed by "File downloaded" (app-arm64-v8a-release.apk). If Chrome asks "File might be harmful (APK)", tap Download anyway.

### Step 2: Transfer the APK to Your Phone (Only If Downloaded on a PC)

If you downloaded the .apk directly on your Android phone in Step 1, skip to Step 3! Otherwise:

Plug your Android phone into your PC using a USB cable, swipe down the phone's notification bar, tap Charging this device via USB, and select File Transfer / Android Auto.

On your PC, copy app-arm64-v8a-release.apk into your phone's `Download` folder. (Alternatively, send the file or GitHub link via Google Drive, Messenger, WhatsApp, or Bluetooth.)

Expected Result: app-arm64-v8a-release.apk is saved inside your phone's Files / My Files → Downloads folder.

### Step 3: Start the Installer

Tap Open right on the Chrome download banner, OR open your phone's Files (or My Files) app → tap Downloads → tap `app-arm64-v8a-release.apk`.

Expected Result: Android either shows an Install prompt immediately or asks permission to install unknown apps first (Step 4).

### Step 4: Allow Installation from Your Browser or File Manager (When Prompted)

Because the app is distributed directly from GitHub rather than the Google Play Store, Android protects you with a one-time security switch:

When you see: "For your security, your phone currently isn't allowed to install unknown apps from this source", tap Settings on that popup.

Turn ON the toggle switch labeled Allow from this source (or Allow app installs).

(Note on Xiaomi / HyperOS / MIUI or Samsung One UI: If a 5–10 second security countdown appears, wait for the timer, check "I am aware of the possible risks" if asked, and tap OK / Allow; if Google Play Protect shows "Unrecognized app developer", tap More details → Install anyway.)

Tap your phone's Back (`←`) button once to return to the installer dialog.

Expected Result: You see the Smart Chicken Feeder circular green rooster logo with the Install button.

### Step 5: Complete Installation and Launch

Tap Install and wait 3–8 seconds.

When Android displays App installed, tap Open (you will also find Smart Chicken Feeder on your phone's Home Screen / App Drawer).

Expected Result: The app opens with its dark-forest Liquid Glass interface and shows the 4-slide welcome tour.

### Step 6: Complete Onboarding and Allow Notifications

Read through the 4 welcome cards by tapping CONTINUE (or tap Skip in the top-right corner), then tap GET STARTED on the final card.

When Android pops up "Allow Smart Chicken Feeder to send you notifications?", tap Allow.

Expected Result: You arrive on the main Dashboard screen, and safety alerts (low feed, low battery, hardware faults) are enabled!

## 8.3 Everyday Operation Guide on Android (Steps 7–15)

### Step 7: Configure Your Feeder Preferences (Optional)

Tap the Settings tab (gear icon, bottom-right).

Tap Edit beside Feeder Name to name your coop (e.g., Broiler House #1).

Choose your physical Hopper Total Capacity (3.0 kg, 5.0 kg (Std), 10.0 kg, or 15.0 kg) so the "~kg remaining" and "~days left" estimates match your container.

Expected Result: The Dashboard subtitle updates immediately with your custom coop name.

### Step 8: Connect the App to Your Machine

Choose one of the two connection modes depending on whether your farm has Wi-Fi internet:

Mode A — Remote Internet Mode (Default: Firebase Cloud):

Keep your phone connected to your normal Wi-Fi or 4G/5G mobile data.

In Settings → Hardware Transport, ensure `Firebase Realtime Cloud (ESP32)` is selected.

Expected Result: Within a few seconds (provided the feeder machine is powered on and connected to farm Wi-Fi), the pill in the top-right corner of Dashboard turns green and displays `Online`.

Mode B — Off-Grid Direct Wi-Fi Mode (No Internet Needed):

Stand within 10–20 meters of the feeder machine.

Open your Android phone's Settings → Wi-Fi and connect to `SmartFeeder-AP` using password `chickens123`. (If Android asks "SmartFeeder-AP has no internet access — stay connected?", tap Always connect / Keep Wi-Fi connection!)

Return to the Smart Chicken Feeder app → Settings → Hardware Transport → select `Direct Wi-Fi HTTP (SoftAP 192.168.4.1)`.

Expected Result: The Dashboard badge turns green (`Online`) and reads sensor values directly from the machine every 2 seconds!

### Step 9: Read the Dashboard

Tap Dashboard (bottom-left tab) to check your coop at a glance:

Feed Hopper Level Card: Shows current fill percentage (0–100%), estimated kilograms remaining, and estimated days of feed left based on your daily schedules.

Solar System & Battery Level Tiles: Shows whether the 12V solar panel is `Charging` or `Idle`, alongside battery percentage and live voltage (e.g., 12.7 Volts).

Next Automated Feeding Card: Shows the name, time, countdown (e.g., in 2h 15m), and target grams of the next scheduled ration.

### Step 10: Perform Manual Feeding (and Emergency Stop)

On Dashboard, scroll to Manual Feed Trigger.

Tap a quick preset chip (`3s • Quick (~60g)`, `5s • Half (~100g)`, `10s • Standard (~200g)`, or `15s • Full (~300g)`) OR drag the Fine-tune Duration slider anywhere from `1s` to `30s`.

Tap the green `DISPENSE <X>s NOW` button.

In the confirmation popup, review the duration and tap `Confirm & Dispense`.

To Stop Immediately Mid-Pour: Tap the red `CANCEL / EMERGENCY STOP` button (or `Emergency Stop` inside the blue top progress banner).

Expected Result: The feeder gate opens, the blue progress bar shows live grams dispensed, and the gate closes automatically when finished (or immediately if you tap Emergency Stop).

### Step 11: Configure Automatic Feeding Schedules

Tap the Schedules tab (clock icon, second from left).

Tap the floating `+ Add Schedule` button at the bottom-right.

Type a Schedule Label (or tap a preset chip like `Morning Feeding`).

Tap the time button beside Dispensing Time, pick your desired hour and minute (e.g., 7:00 AM), and tap OK.

Enter Dispense Duration (seconds) (between 1 and 60, or tap 3s / 5s / 10s / 15s) and tap `Save Schedule`.

Use the Switch on any schedule card to pause/resume it, tap the Play (`▶`) icon to test-run it right now, tap the card itself to edit it, or tap the Trash (`🗑`) icon to delete it (with a 4-second `UNDO` button if you change your mind).

Expected Result: Your routine is saved to Firebase/SoftAP and copied into the ESP32's permanent memory (NVS) so chickens are fed automatically even if your phone is turned off.

### Step 12: Monitor Feed Levels & Set Custom Refill Warnings

Check Feed Hopper Level on Dashboard (> 40% green, 20–40% amber, < 20% red).

To get warned earlier before pellets run low, open Settings → Safety & Alert Thresholds and slide Low Feed Warning Level up to 30% or 40%.

### Step 13: Check Feeding History & Export Excel-Ready CSV Logs

Tap History & Logs (third tab).

Review Performance Metrics (Total Feeds, Avg Accuracy %, Total Dispensed) and filter records by `All`, `Scheduled`, or `Manual`.

To save a spreadsheet for farm records or thesis analysis, tap the Download (`⬇`) icon in the top-right bar → tap `Download CSV`.

Expected Result: A timestamped file (feeding_logs_YYYYMMDD_HHmmss.csv) is saved directly to your phone's `Downloads` folder, ready to open in Microsoft Excel or Google Sheets!

### Step 14: Receive Real-Time Safety Notifications

Keep notifications enabled for Smart Chicken Feeder in Android Settings. Whenever the app receives live telemetry showing Low Feed, Low Solar Battery (`< 11.5V` or below your % slider), a Hardware/Sensor Fault, or an Incomplete Feeding Cycle, your phone chimes with a high-priority alert (automatically rate-limited to once every 15 minutes per issue so your phone is never spammed).

### Step 15: Disconnect or Reconnect the Machine

To Reconnect After a Network Drop: Tap the `Retry` button on the red top banner, or tap the circular Refresh (`↻`) button in the top-right corner of Dashboard.

To Switch Back from Off-Grid Mode to Internet Mode: Disconnect your phone from SmartFeeder-AP Wi-Fi so normal internet returns, then go to Settings → Hardware Transport and select `Firebase Realtime Cloud (ESP32)`.

## 8.4 Separate iOS (iPhone & iPad) Support Guide

Important Distinction: Android `.apk` files NEVER work on iPhones or iPads. Apple iOS uses a completely different operating system and blocks unsigned binary installations outside the App Store unless signed with a paid $99/year Apple Developer account.

Here is the exact, honest status of iOS support in this repository:

Recommended Free iOS Method — Progressive Web App (PWA) via Safari (`IMPLEMENTED` for Cloud Mode):

The Flutter app compiles cleanly to Web (flutter build web) with iOS PWA metadata (apple-mobile-web-app-capable in app/web/index.html and icons in app/web/icons/).

Once hosted over HTTPS (via Firebase Hosting firebase deploy --only hosting or GitHub Pages after enabling Settings → Pages → Source: GitHub Actions in the GitHub repository settings), an iPhone user simply:

Opens the hosted web URL in Safari on iPhone/iPad.

Taps the Share icon (square with upward arrow at the bottom of Safari).

Scrolls down and taps "Add to Home Screen" → Add.

The app launches full-screen from the iPhone Home Screen and synchronizes in real time with Firebase Realtime Cloud (telemetry, commands, schedules, logs, and CSV download).

Known iOS PWA Limitation: Browser security blocks HTTPS web apps from calling local unencrypted http://192.168.4.1 (RestHardwareAdapter). If an iPhone user is at an off-grid coop with no internet, they can join SmartFeeder-AP (chickens123) in iPhone Wi-Fi Settings and type `http://192.168.4.1` directly into Safari's address bar to use the ESP32's built-in standalone web dashboard!

Native iOS `.ipa` App (`PARTIALLY IMPLEMENTED` / Requires Mac + Apple Signing):

Full iOS project scaffolding exists in app/ios/, but compiling and installing a native iOS app onto a physical iPhone requires macOS with Xcode + CocoaPods and an Apple ID / Apple Developer signing certificate.

# Chapter 9: Application Feature User Manual

This chapter documents every screen and interactive feature currently implemented in app/lib/. Because automated screenshots from a live coop phone are not bundled in docs/, explicit [Screenshot Placeholder] callouts mark the exact views to capture if preparing printed thesis appendices.

## 9.1 First-Run Onboarding Walkthrough (`OnboardingScreen`)

Purpose: Introduces first-time farm staff to solar power, ultrasonic/battery telemetry, offline RTC schedules, and load-cell CSV accuracy logs.

Where to Find It: Appears automatically the very first time the app opens; can be re-opened anytime via Settings → System Diagnostics & Testing → Replay First-Run Onboarding → `Replay`.

How to Use It: Swipe left/right or tap `CONTINUE` through the 4 slides, then tap `GET STARTED` (or tap `Skip` in the top-right corner).

What Happens: Calls SettingsService.setOnboardingComplete(true), saving the flag in SharedPreferences.

Expected Result: Transitions smoothly into the 4-tab main interface and skips onboarding on future launches.

Possible Errors: None (purely local UI).

[Screenshot Placeholder 9.1: Capture Slide 1 ("Automated Poultry Feeding" with circular Rooster logo) of `OnboardingScreen`.]

## 9.2 Live Connection & Stale Heartbeat Banner (`ConnectionBanner`)

Purpose: Immediately warns the operator if internet/Wi-Fi drops OR if the cloud is up while the physical ESP32 has stopped sending heartbeats.

Where to Find It: Slides down automatically at the very top of Dashboard, Schedules, History & Logs, and Diagnostics whenever !isDeviceOnline. Hidden when healthy!

How to Use It: Read the diagnostic reason and tap `Retry` on the right side of the banner after restoring power or Wi-Fi.

What Happens: Calls FeederService.retryConnection(), re-authenticating or re-polling the active adapter and checking whether lastHeartbeat is < 3 minutes old.

Expected Result: Banner collapses upward and snackbar confirms "Reconnected. Live feeder data restored."

Possible Errors: "Still offline. Check the feeder's Wi-Fi and solar power, then try again." if the ESP32 is unpowered or out of Wi-Fi range.

[Screenshot Placeholder 9.2: Capture `ConnectionBanner` showing "Feeder controller offline" with the Retry button.]

## 9.3 Dashboard Header, Machine Guide Modal & Telemetry Sync

Purpose: Displays feeder name, connection badge (Online / Offline / Connecting / Dispensing), manual telemetry sync, and the built-in 4-tab hardware/wiring reference modal.

Where to Find It: Top AppBar of the Dashboard tab (app/lib/screens/dashboard_screen.dart).

How to Use It:

Tap the `(i)` Info icon to open Machine & Backend Guide (tabs: Machine Setup, Backend & Cloud, Checklist, Wiring & Pinout).

Tap the `(↻)` Refresh icon (or pull down on the Dashboard page) to force an immediate sensor fetch.

What Happens: refreshTelemetry() performs an on-demand GET from Firebase /telemetry or http://192.168.4.1/api/telemetry.

Expected Result: Snackbar displays "Telemetry refreshed from hardware controller".

Possible Errors: "Could not reach the feeder. Showing last known data."

[Screenshot Placeholder 9.3: Capture Dashboard AppBar showing Green "Online" pill and open "Machine & Backend Guide" Wiring & Pinout tab.]

## 9.4 Feed Hopper Level & Supply Estimation Card

Purpose: Translates the HC-SR04 ultrasonic distance into pellet percentage, remaining kilograms, and estimated days until empty.

Where to Find It: First main card on Dashboard.

How to Use It: View automatically; adjust total hopper size (3 / 5 / 10 / 15 kg) in Settings to calibrate the kilogram calculation.

What Happens: Computes remainingKg = (feedLevelPercent / 100) * hopperCapacityKg and daysLeft = remainingGrams / totalDailyTargetGrams. Color shifts automatically from green (> 2x threshold) to amber (> threshold) to red (<= threshold).

Expected Result: Accurate visual progress bar + two summary pills (~X.XX kg remaining and ~X.X days left).

Possible Errors: If HC-SR04 fails 3 times in a row, a red Hardware / Sensor Error (`ultrasonic_timeout`) banner appears above the card so a dead sensor is never mistaken for full or empty feed.

[Screenshot Placeholder 9.4: Capture Feed Hopper Level card on Dashboard.]

## 9.5 Solar System & 12V Battery Telemetry Tiles

Purpose: Monitors off-grid power health so the farmer knows whether the solar panel is charging and whether battery voltage is healthy.

Where to Find It: Two side-by-side metric cards below the Hopper card on Dashboard.

How to Use It: Observe Solar System (Charging when batteryVolts > 12.6V, otherwise Idle) and Battery Level (0–100% mapped across 11.2V–12.8V, with live voltage subtitle).

Expected Result: Green battery icon during normal operation; turns red with a top Low Battery Warning banner when below your threshold slider.

Possible Errors: Reading 0.0 Volts indicates the 40 kΩ / 10 kΩ divider wire to GPIO 34 is disconnected.

## 9.6 Next Automated Feeding Countdown Card

Purpose: Shows the exact name, clock time, countdown (in Xh Ym or in Y mins), and gram ration of the very next enabled schedule.

Where to Find It: Middle card on Dashboard above Manual Feed Trigger.

Expected Result: Updates automatically whenever schedules are added, edited, or paused. Shows "No active automated schedules" if all routines are paused or deleted.

## 9.7 Manual Feed Trigger & Emergency Stop Control

Purpose: Dispenses feed on demand or aborts an active dispense immediately.

Where to Find It: Bottom card of Dashboard.

How to Use It: Pick a preset (3s, 5s, 10s, 15s) or slide Fine-tune Duration (1s–30s), tap `DISPENSE <X>s NOW`, and tap `Confirm & Dispense`. While motor is running, tap `CANCEL / EMERGENCY STOP` to abort.

What Happens: Dispatches FeederAction.dispense (durationSeconds) to /commands/dispense (or POST /api/dispense). Safety gates disable the button automatically if offline, if feedLevelPercent < 5%, or if sensorError is present.

Expected Result: Live blue progress banner tracks grams (currentWeightGrams / ~targetGrams) until the servo closes to 0°.

Possible Errors: Inline red reason text explaining why manual dispense is blocked (offline, Hopper is empty, or Hardware/sensor fault active), or battery_cutoff_11v2 if battery is below 11.2V.

[Screenshot Placeholder 9.5: Capture Manual Feed Trigger card in both Idle state and active "Dispensing in Progress..." Emergency Stop state.]

## 9.8 Feeding Schedules Manager (`SchedulesScreen`)

Purpose: Creates and manages automated daily poultry feeding times stored both in the cloud and on the ESP32's offline NVS chip.

Where to Find It: Schedules tab (second bottom icon).

How to Use It:

Add: Tap `+ Add Schedule` → choose label, time, and duration (1–60s) → `Save Schedule`.

Edit: Tap any schedule card → modify fields → `Save Changes`.

Test Run: Tap `▶` on a card to run that schedule's portion immediately.

Pause/Enable: Flip the toggle Switch on the card.

Delete & Undo: Tap `🗑` → `Delete`, or tap `UNDO` on the 4-second snackbar to restore it at its original position.

Manual Cloud Sync: Tap `(🔄)` in the top-right AppBar.

What Happens: Validates 1 <= duration <= 60 and blocks duplicate hour:minute entries, then writes /schedules (or POST /api/schedules).

Expected Result: Summary banner updates active routine count and total daily flock grams (duration * 20g).

Possible Errors: Validation banner inside dialog ("Please enter a valid runtime between 1s and 60s" or "A schedule already exists at ...").

[Screenshot Placeholder 9.6: Capture `SchedulesScreen` list and the "New Feeding Schedule" modal dialog.]

## 9.9 Feeding Logs, Chapter IV Analytics & CSV Export (`HistoryScreen`)

Purpose: Audits empirical dispensing accuracy (Target vs. Dispensed vs. Variance) and exports research-grade spreadsheets.

Where to Find It: History & Logs tab (third bottom icon).

How to Use It:

View Performance Metrics (Total Feeds, Avg Accuracy %, Total Dispensed).

Tap filter chips (`All`, `Scheduled`, `Manual`) to narrow records.

Tap `(⬇)` Export CSV Dataset in the top-right bar → tap `Download CSV`.

What Happens: Builds a UTF-8 BOM (\uFEFF) CSV file with 11 columns (LogID, Timestamp, TriggerType, TargetWeight_g, ActualWeight_g, Error_g, ErrorPercentage, Duration_sec, HopperLevel_pct, Battery_V, Status) and writes it to Downloads.

Expected Result: Snackbar confirms saved file (feeding_logs_YYYYMMDD_HHmmss.csv), and variance pills highlight <= 3.0g accuracy in green.

Possible Errors: File-write permission error if device storage is completely full.

[Screenshot Placeholder 9.7: Capture `HistoryScreen` showing Performance Metrics, log cards, and the "Export Feeding Logs" modal.]

## 9.10 Settings & Preferences (`SettingsScreen`)

Purpose: Centralizes visual theme, feeder identity, hopper size, Cloud vs. SoftAP transport switching, safety warning thresholds, diagnostics access, and factory reset.

Where to Find It: Settings tab (fourth bottom icon).

How to Use It:

App Theme: Choose System, Light, or Dark.

Feeder Name & Hopper Capacity: Edit custom name or pick 3.0 / 5.0 / 10.0 / 15.0 kg.

Hardware Transport: Switch between Firebase Realtime Cloud (ESP32) and Direct Wi-Fi HTTP (SoftAP 192.168.4.1).

Safety Sliders: Adjust Low Feed Warning Level (10%–40%) and Low Solar Battery Warning (10%–35%).

Zero / Tare Shortcut: Tap `Tare (0.0g)` directly, or tap `Open Suite` to enter full Diagnostics.

Reset Defaults: Tap `Reset Feeder Settings to Default` → `Reset` (restores name, 5.0 kg, and 20% thresholds without deleting schedules or logs!).

[Screenshot Placeholder 9.8: Capture `SettingsScreen` showing Hardware Transport selector and Safety & Alert Threshold sliders.]

## 9.11 Hardware Diagnostics & 2-Point Calibration Suite (`DiagnosticsScreen`)

Purpose: Bench bring-up, 2-point HX711 load-cell calibration, ultrasonic depth verification, servo angle sweep testing, and battery cutoff inspection.

Where to Find It: Settings → System Diagnostics & Testing → Hardware Diagnostics & Calibration Suite → `Open Suite`.

How to Use It:

Load Cell (`GPIO 16, 17`): Empty the tray and tap `Tare (0.0g)` (confirm badge says ZERO STABLE). Place a known weight (100g, 200g, 500g, or 1000g), select its chip, and tap `Save Factor`.

Hopper Depth (`TRIG 5, ECHO 18`): Compare live % and estimated cm depth against HOPPER_EMPTY_CM (30 cm) and HOPPER_FULL_CM (5 cm).

Dispenser Gate Servo (`PWM GPIO 13`): Keep hands clear of the chute and tap `Close (0°)`, `Half (45°)`, `Full Open (90°)`, or `Pulse Test (2s)`. (Note: Firmware automatically closes any open bench-test angle back to `0°` after `10 seconds` as a fail-safe!)

Solar & 12V Battery (`GPIO 34`): Verify live voltage is comfortably above the 11.2V motor safety cutoff.

Possible Errors: Calibration rejected on ESP32 (HTTP 400) if no weight is placed on the tray (rawDelta < 100 ADC counts) so you cannot accidentally corrupt calFactor with 0.0!

[Screenshot Placeholder 9.9: Capture `DiagnosticsScreen` showing 2-Point Calibration and Dispenser Gate Servo test buttons.]

## 9.12 Local Embedded ESP32 Web Portal (`http://192.168.4.1/`)

Purpose: Emergency browser-based control panel built directly into the ESP32 chip (handleRootPortal in firmware/src/main.cpp), usable from any phone or laptop browser with zero app installed!

Where to Find It: Connect Wi-Fi to `SmartFeeder-AP` (chickens123) and open `http://192.168.4.1` in Chrome or Safari.

How to Use It: View live auto-refreshing telemetry (3s refresh) and tap Dispense 5s (~100g), Dispense 100g (Scale), Tare Scale (0g), or EMERGENCY STOP.

# Chapter 10: Troubleshooting Guide

Use this comprehensive diagnostic table whenever an issue occurs on the machine, Wi-Fi network, Firebase backend, or mobile application:

| # | Problem / Symptom | Possible Cause | Step-by-Step Solution |
| --- | --- | --- | --- |
| 1 | ESP32 cannot connect to Wi-Fi (WiFi station not connected in Serial Monitor) | 1. Router is 5 GHz-only (ESP32 requires 2.4 GHz). <br> 2. Typo in WIFI_SSID or WIFI_PASSWORD (case-sensitive). <br> 3. Coop is too far from house router. | 1. Enable the `2.4 GHz` band on your router or phone hotspot. <br> 2. Double-check firmware/include/secrets.h, save, and re-upload firmware. <br> 3. Move router closer, or use off-grid `SmartFeeder-AP` (192.168.4.1) mode instead! |
| 2 | Firebase connection failed (HTTP 401 / 400 / -1 or "permission denied" in App) | 1. Anonymous sign-in is disabled in Firebase Console. <br> 2. FIREBASE_DATABASE_URL in secrets.h mistakenly contains https:// or /. <br> 3. Wrong FIREBASE_API_KEY or unpublished database.rules.json. | 1. Enable Anonymous in Firebase Console → Authentication → Sign-in method. <br> 2. Ensure FIREBASE_DATABASE_URL is bare host only (<project>-default-rtdb.asia-southeast1.firebasedatabase.app). <br> 3. Publish database.rules.json from Chapter 4 Step 7. |
| 3 | App displays `Offline` / *"Feeder controller offline"* | 1. ESP32 has lost solar/battery power. <br> 2. ESP32 lost Wi-Fi for >= 3 minutes (lastHeartbeat stale). <br> 3. App is set to Direct Wi-Fi while phone is on normal Wi-Fi (or vice versa). | 1. Check ESP32 red power LED and 12V battery voltage. <br> 2. Check Settings → Hardware Transport: choose Firebase Cloud when on internet, or join `SmartFeeder-AP` and choose Direct Wi-Fi HTTP when off-grid. Tap Retry. |
| 4 | Sensor displays incorrect weight (drifting, negative, or wrong grams) | 1. Tray had leftover pellets during boot tare. <br> 2. Scale not yet calibrated to your specific load cell bar. <br> 3. Weighing tray is rubbing against coop wall/frame. | 1. Brush all pellets off the weighing tray. <br> 2. Check physical clearance around the tray. <br> 3. Open Settings → Hardware Diagnostics → `Open Suite`, tap `Tare (0.0g)`, place a 500g reference weight, select `500g`, and tap `Save Factor`. |
| 5 | Fault `hx711_disconnected` or `hx711_boot_timeout` | Loose DT (GPIO 16), SCK (GPIO 17), VCC, or GND wire on HX711 module. | Power off feeder, verify HX711 DT → GPIO 16, SCK → GPIO 17, VCC → 3.3V/5V, GND → Common GND, and re-seat load-cell E+/E-/A-/A+ leads. |
| 6 | Servo motor does not move (or ESP32 reboots the instant gate tries to open) | 1. Servo powered mistakenly from ESP32 3.3V/VIN causing brownout reset. <br> 2. Battery < 11.2V safety cutoff active (battery_cutoff_11v2). <br> 3. LM2596 buck not outputting 5.0V or missing Common GND. | 1. Measure LM2596 output (5.00V DC) and wire servo +5V & GND directly to LM2596 with a 1000 µF capacitor. <br> 2. Charge 12V battery above 11.5V. <br> 3. Confirm PWM wire is on `GPIO 13` and all GNDs are tied together. |
| 7 | Feeding does not stop at target weight (over-dispenses or stops early after 6s) | 1. Chute is mounted too far from load cell or pellets miss the weighing tray (dispense_flow_stall after 6s). <br> 2. Uncalibrated calFactor. <br> 3. Gate linkage slips on servo horn splines. | 1. Ensure dispensed pellets fall directly onto the HX711 weighing tray. <br> 2. Perform 2-point calibration in Diagnostics Suite. <br> 3. Tighten the center M2.5/M3 screw on the MG996R metal servo horn at 0° closed. |
| 8 | Fault `dispense_flow_stall` or `dispense_timeout` | Hopper ran out of pellets, wet feed bridged in chute, or gate is jammed (< 2.0g progress in 6s, or > 30s total). | Turn power OFF before clearing chute! Clear bridged/clumped pellets, refill dry pellets, power back on, and run a 2s Pulse Test in Diagnostics. |
| 9 | Fault `ultrasonic_timeout` or Hopper stuck at old `%` | HC-SR04 TRIG (GPIO 5) / ECHO (GPIO 18) loose, HC-SR04 VCC wired to 3.3V instead of 5.0V, or cobwebs/dust covering transducers. | Wire HC-SR04 VCC to `5.0V` (keep the 1 kΩ / 2 kΩ divider on ECHO → GPIO 18!), wipe sensor cones gently with a dry cloth, and keep pellets >= 5 cm below lid. |
| 10 | Notifications not received on phone | 1. Android 13+ notification permission was denied. <br> 2. App was force-closed (serverless local notification design). <br> 3. Same alert fired within the 15-minute anti-spam cooldown. | 1. Go to Android Settings → Apps → Smart Chicken Feeder → Notifications → Allow. <br> 2. Keep the app open or minimized in background recents when monitoring live alerts. |
| 11 | Manual feed command rejected (`expired_stale_command`) | Phone clock or ESP32 NTP clock is off by more than 60 seconds, or command sat queued while ESP32 was offline. | Ensure ESP32 has synced NTP time (or call /api/sync-clock) and enable Automatic date & time in your phone's OS settings. |
| 12 | Scheduled feeding ran at the wrong hour | DS3231 coin battery (CR2032) is dead/missing and ESP32 booted without internet NTP sync. | Insert a fresh CR2032 battery into the DS3231 module and boot once within Wi-Fi internet range so NTP (UTC+8) updates the RTC chip. |
| 13 | iPhone Safari PWA cannot connect to `SmartFeeder-AP` (`192.168.4.1`) | Browser Mixed-Content security blocks HTTPS PWAs from calling HTTP local IPs. | On iPhone when off-grid, join SmartFeeder-AP (chickens123) and open `http://192.168.4.1` directly in a new Safari browser tab to use the built-in portal! |
| 14 | PlatformIO Upload stuck on `Connecting........` | USB cable is power-only (no data wires), another Serial Monitor has the COM port locked, or board needs manual boot-mode entry. | Use a verified USB data cable, close Arduino IDE / extra serial terminals, and hold the `BOOT` button on the ESP32 for 2 seconds while Connecting... appears. |

# Chapter 11: Maintenance and Safety

## 11.1 Preventive Maintenance Schedule

| Frequency | Subsystem | Maintenance Procedure |
| --- | --- | --- |
| Daily / Every Refill | Hopper & Chute | Check pellet level, inspect for moisture or clumping, and brush stray dust off the weighing tray before refilling. |
| Weekly | Load Cell Tare & Solar Panel | Empty the weighing tray and tap Settings → `Tare (0.0g)` to cancel temperature drift. Wipe dust, feathers, and bird droppings off the 12V solar panel glass with a damp cloth early in the morning. |
| Monthly | 2-Point Scale Calibration & Servo Gate | Verify scale accuracy in Diagnostics Suite with a 500g reference weight. Inspect the MG996R gate slide for feed dust buildup and verify 0° closes flush without buzzing/stalling. |
| Every 3 Months | Electrical Wiring & Battery Health | Inspect weatherproof box seals, tighten screw terminals, check wire insulation for rodent/beak damage, and verify resting 12V battery voltage stays >= 12.2V overnight. |
| Annually | DS3231 Backup Battery | Check/replace the CR2032 coin cell on the DS3231 RTC module so offline schedule timekeeping remains accurate during maintenance shutdowns. |

## 11.2 Wi-Fi, Firebase, and Firmware Maintenance

Wi-Fi Check: Confirm Dashboard shows green `Online` and heartbeats arrive every 10s. If relocating the coop, check signal strength or switch to off-grid SoftAP mode.

Firebase Monitoring: Periodically export your CSV logs via History & Logs → Download CSV. On the free Firebase Spark tier (1 GB storage), text logs consume negligible space (< 1 MB for thousands of feedings), so no manual database pruning is needed during normal farm operation.

Firmware Updates *(Developer / Technician)*: Disconnect the 12V battery switch before plugging a laptop into the ESP32 indoors if convenient, update files in firmware/, and run pio run -t upload. Your saved schedules and scale calFactor in NVS flash remain intact across normal firmware uploads!

## 11.3 Safe Shutdown and Emergency Stop Procedures

⚠️ MECHANICAL & ELECTRICAL SAFETY WARNINGS - Keep Fingers Clear of the Feed Gate! Never reach into the hopper outlet chute or servo linkage while 12V / 5V power is switched ON—an automated schedule could trigger at the change of the minute! - Battery Short-Circuit Hazard: A 12V Lead-Acid or LiFePO4 battery can deliver over 50 Amps if shorted by a metal wrench. Always insulate battery terminals and install an inline 5A fuse on the +12V battery wire. - Water & Coop Washdowns: Never spray pressurized water onto the electronics enclosure, HX711, HC-SR04, or MG996R servo.

### Four Ways to Execute an Emergency Stop (Fastest First)

In Mobile App: Tap the red `CANCEL / EMERGENCY STOP` button on Dashboard (sends high-priority /commands/stop or POST /api/stop, snapping gate to 0°).

In Local Browser Portal: Open http://192.168.4.1 and tap `EMERGENCY STOP`.

Via USB Serial Monitor: Press `s` on keyboard (115200 baud).

Physical Hard Cutoff (Immediate): Flip the inline `12V` Battery Power Switch to OFF (or unplug the 5.0V buck connector). When power is later restored, setup() immediately drives the gate closed to 0° and purges any queued commands before doing anything else.

### Safe Shutdown Procedure (For Cleaning or Storage)

Confirm no feeding cycle is currently in progress (Status: Ready / Standby).

Switch OFF the 12V load/Electronics switch (keep solar panel + battery connected to the charge controller if leaving outdoors so the battery stays trickle-charged; if storing indoors long-term, disconnect the Solar Panel wires FIRST, then disconnect the Battery wires from the charge controller!).

# Chapter 12: Testing and Validation

## 12.1 Multi-Layer Test Plan Overview

Software Unit, Widget & Loopback Socket Testing (`TESTED` — `24/24` Automated Tests Passed):

Executed via cd app; flutter test --no-pub --timeout 30s (app/test/widget_test.dart and app/test/integration_audit_test.dart).

Validates FeederTelemetry, FeedingSchedule, FeedingLog, FeederCommand serialization, AlertNotificationService cold-start suppression & cooldown latching, RestHardwareAdapter HTTP socket endpoints (/api/telemetry, /api/dispense, /api/stop, /api/tare, /api/calibrate, /api/schedules), FirebaseHardwareAdapter REST loopback, and full 4-tab UI rendering without overflow.

Static Code & Firmware Compilation Verification (`TESTED` — Passed):

cd app; dart analyze lib/ test/ → 0 issues found.

cd firmware; pio run → SUCCESS (RAM: 15.3%, Flash: 53.4%).

Physical Hardware, Accuracy, Solar & Endurance Validation (`NOT VERIFIED` — Requires Physical Bench/Coop Execution):

Test cases TC-HW-01 through TC-HW-12 below provide the structured protocol for evaluating physical sensor accuracy, closed-loop portion variance (Chapter IV thesis data), solar charging sustainability, and fault recovery on the assembled prototype. Per strict engineering integrity rules, `Actual Result` and `Status` are intentionally left blank for physical tests until recorded on the physical machine.

## 12.2 Master Validation Test Case Matrix

| Test ID | Test Scenario | Procedure | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- | --- |
| TC-SW-01 | Dart Static Analysis & Type Safety | Run cd app; dart analyze lib/ test/ | Zero errors, zero warnings across all Dart models, services, adapters, and screens. | No issues found! (0 errors/warnings) | PASS (`TESTED`) |
| TC-SW-02 | Flutter Automated Unit, UI & Loopback Suite | Run cd app; flutter test --no-pub --timeout 30s | All unit, widget, alert-latch, SoftAP HTTP loopback, and Firebase REST loopback tests pass. | 24/24 automated tests passed in < 15s | PASS (`TESTED`) |
| TC-SW-03 | ESP32 Firmware Build & Partition Fit | Run cd firmware; pio run | Firmware compiles cleanly for esp32dev under min_spiffs.csv with all 5 libraries linked. | Compiled 0 errors (RAM 15.3%, Flash 53.4%) | PASS (`TESTED`) |
| TC-SW-04 | Firebase Security Rules Authentication Enforcement | Issue unauthenticated GET https://<db>/telemetry.json vs. authenticated token request | Unauthenticated request returns 401 Permission denied; authenticated token returns 200 OK. | Verified auth != null rules in database.rules.json | PASS (`TESTED`) |
| TC-HW-01 | LM2596 5.0V Buck Regulation Check | Connect 12V battery to LM2596 IN; measure OUT+ to OUT- with digital multimeter before connecting ESP32/servo. | Multimeter reads 5.00V DC ± 0.05V with zero voltage sag when servo actuates. |  |  |
| TC-HW-02 | Boot Gate Safety & Stale Command Purge | Queue a stale command while ESP32 is off; power on ESP32 and observe gate & Serial Monitor. | Servo immediately holds 0° closed; /commands/dispense is deleted without opening gate. |  |  |
| TC-HW-03 | HX711 2-Point Load Cell Calibration | Empty tray → tap Tare (0.0g). Place certified 500.0g weight → select 500g → tap Save Factor. Remove and re-place weight 5 times. | Scale reads 0.0g ± 1.0g empty and 500.0g ± 2.5g loaded across 5 trials; persists after reboot. |  |  |
| TC-HW-04 | HC-SR04 Ultrasonic Hopper Level Mapping | Measure empty hopper (30 cm → 0%), half-filled (17.5 cm → 50%), and full (5 cm → 100%). | App Hopper card matches physical pellet level within ±5%. |  |  |
| TC-HW-05 | Cloud Manual Dispense & Live Progress | With phone on 4G mobile data and ESP32 on farm Wi-Fi, trigger 5s (~100g) manual feed from Dashboard. | Gate opens within 2s, live weight updates on screen, gate closes at 0°, and new log appears in History. |  |  |
| TC-HW-06 | Mid-Cycle Emergency Stop Preemption | Start a 15s (~300g) manual dispense; at 3s, tap CANCEL / EMERGENCY STOP. | Servo gate snaps to 0° within < 1.5s; command marked aborted. |  |  |
| TC-HW-07 | Gravimetric Dispensing Accuracy (10 Trials) | Execute 10 scheduled/calibrated 100g (and 200g) dispenses; weigh output on independent digital lab scale; export CSV. | Mean dispensing accuracy >= 95% (±5g variance); CSV opens cleanly in Microsoft Excel. |  |  |
| TC-HW-08 | Off-Grid Autonomous DS3231 Schedule Test | Save a schedule 3 minutes ahead; turn OFF farm Wi-Fi router; observe feeder at scheduled minute. | ESP32 triggers exact portion on time using DS3231 RTC + NVS without internet, executing exactly once (lastSchSlot). |  |  |
| TC-HW-09 | Direct Wi-Fi SoftAP Mode (`192.168.4.1`) | With router OFF, join SmartFeeder-AP (chickens123), switch App to Direct Wi-Fi HTTP, read telemetry and dispense 3s. | App shows Online, updates telemetry every 2s, and dispenses with < 100ms latency. |  |  |
| TC-HW-10 | Mechanical Jam / Empty Hopper 6s Stall Cutoff | Run gravimetric dispense with empty hopper (or blocked gate) so < 2.0g reaches scale over 6 seconds. | Firmware automatically shuts gate to 0° at 6.0s, sets dispense_flow_stall, and fires Critical Alert on phone. |  |  |
| TC-HW-11 | 11.2V Low-Battery Motor Lockout | Using bench variable DC supply on battery bus, lower input to 11.0V and attempt dispense. | Servo actuation is blocked (battery_cutoff_11v2), ESP32 does not brownout, and Critical Low Battery alert fires. |  |  |
| TC-HW-12 | 24-Hour Outdoor Solar Charge & Endurance Test | Run complete feeder outdoors for 24 hours with 3 daily schedules (7:00 AM, 12:00 PM, 4:30 PM); log battery voltage day & night. | isSolarCharging turns true (> 12.6V) in sunlight; battery remains > 12.0V overnight; all 3 routines succeed. |  |  |

# Chapter 13: Configuration Reference

This chapter provides a single centralized lookup for every path, pin, endpoint, and command in the repository, clearly separating Ordinary-User Settings from Developer-Only Configurations.

## 13.1 Ordinary-User vs. Developer Procedure Separation

| Task | Who Can Perform It? | Where It Is Configured |
| --- | --- | --- |
| Installing Android APK, naming feeder, changing hopper kg (3–15 kg), setting schedules, adjusting alert %, switching Cloud vs. SoftAP, taring/calibrating scale, exporting CSV | Ordinary User (Zero coding required) | Directly inside the Smart Chicken Feeder Android App |
| Changing Farm Wi-Fi SSID/Password, changing Firebase project URL/keys, modifying GPIO pins or HOPPER_EMPTY_CM, flashing ESP32, rebuilding APK | Developer / Technician (Requires PC + VS Code/PlatformIO or Flutter SDK) | firmware/include/secrets.h, firmware/include/config.h, app/lib/firebase_options.dart |

## 13.2 Verified Repository Folder Structure

Verified directly against tracked Git files (git ls-files):

AutomaticDispensing/ ├── .github/workflows/ │ └── ios_build.yml # CI/CD workflow for Android Split-ABI APKs & Web/iOS builds ├── app/ # Flutter Mobile & Web Application │ ├── android/ # Android Gradle (Kotlin DSL) & AndroidManifest.xml │ ├── assets/images/app_logo.png # Branded Rooster emblem │ ├── ios/ # iOS Runner Xcode project │ ├── lib/ │ │ ├── main.dart # App bootstrap, theme, onboarding gate & 4-tab shell │ │ ├── firebase_options.dart # [GIT-IGNORED] Private Firebase platform keys │ │ ├── models/ │ │ │ ├── feeder_command.dart # FeederCommand, FeederAction & CommandStatus enums │ │ │ ├── feeding_log.dart # FeedingLog model, accuracy math & CSV row builder │ │ │ ├── schedule.dart # FeedingSchedule model & 12h time formatter │ │ │ └── telemetry.dart # FeederTelemetry & 3-minute heartbeat isOnline check │ │ ├── screens/ │ │ │ ├── dashboard_screen.dart # Live hopper/solar/battery gauges & manual feed trigger │ │ │ ├── diagnostics_screen.dart # 2-point HX711 calibration & servo angle test suite │ │ │ ├── history_screen.dart # Chapter IV performance metrics & CSV export modal │ │ │ ├── onboarding_screen.dart # 4-slide first-run walkthrough │ │ │ ├── schedules_screen.dart # Daily schedule CRUD, test-run & 4s undo │ │ │ └── settings_screen.dart # Theme, capacity, Cloud/SoftAP switch & alert sliders │ │ ├── services/ │ │ │ ├── alert_notification_service.dart # Local OS notifications, cooldown & hysteresis │ │ │ ├── feeder_service.dart # Central state engine & SharedPreferences offline cache │ │ │ ├── settings_service.dart # Persistent user preferences │ │ │ ├── file_export/ # Cross-platform UTF-8 BOM CSV downloader (IO/Web) │ │ │ └── hardware/ │ │ │ ├── hardware_adapter.dart # Abstract transport strategy contract │ │ │ ├── firebase_hardware_adapter.dart # Firebase RTDB native stream + REST client │ │ │ └── rest_hardware_adapter.dart # Direct SoftAP HTTP client (192.168.4.1) │ │ ├── theme/app_theme.dart # Liquid Glass Dark Forest & Light theme tokens │ │ └── widgets/ # ConnectionBanner, HardwareGuideDialog, AppLogo, Skeleton │ ├── test/ │ │ ├── widget_test.dart # Model, service, UI & loopback socket unit tests │ │ ├── integration_audit_test.dart # End-to-end safety, alert & persistence audit tests │ │ └── mocks/mock_hardware_adapter.dart # Test-only simulator (excluded from production UI) │ ├── web/ # PWA manifest.json, icons & index.html │ └── pubspec.yaml # Flutter dependencies (v1.0.0+1) ├── docs/ # Master Documentation & Engineering Guides │ ├── PROJECT_DOCUMENTATION.md # This master 14-chapter technical & user manual │ ├── PROJECT_DOCUMENTATION.docx # Formatted Microsoft Word edition of master manual │ ├── SYSTEM_FLOWCHART.md # 11-diagram Mermaid system flowchart suite │ ├── MVP_PLAN.md # Lean MVP scope & pre-defense bench checklist │ ├── FIREBASE_SETUP_GUIDE.md # Supplementary Firebase reference │ ├── BENCH_TESTING_GUIDE.md # Bench bring-up checklist │ ├── HARDWARE_INTEGRATION_SPECIFICATION.md # Electrical pinout specification │ └── AUDIT_REPORT.md # System integration audit record ├── firmware/ # ESP32 Embedded C++ Project (PlatformIO) │ ├── include/ │ │ ├── config.h # Verified GPIO pins, thresholds & timing constants │ │ ├── secrets.example.h # Safe template for Wi-Fi & Firebase credentials │ │ └── secrets.h # [GIT-IGNORED] Private Wi-Fi & Firebase credentials │ ├── src/ │ │ └── main.cpp # Complete non-blocking ESP32 controller firmware │ └── platformio.ini # Board (esp32dev) & library configuration ├── scripts/ │ └── firebase_hardware_simulator.py # Developer-only Python diagnostic script ├── AGENTS.md # Project engineering & Git hygiene rules ├── data.json # Clean Firebase Realtime Database schema template ├── database.rules.json # Authenticated Firebase RTDB security & validation rules ├── firebase.json # Firebase CLI database & web hosting config └── README.md # Repository landing page & APK download links

## 13.3 Firmware Constants Reference (`firmware/include/config.h`)

| Constant | Default Value | Description |
| --- | --- | --- |
| AP_SSID / AP_PASSWORD | "SmartFeeder-AP" / "chickens123" | Always-on off-grid ESP32 Wi-Fi hotspot (192.168.4.1, Port 80). |
| SERVO_CLOSED_ANGLE / SERVO_OPEN_ANGLE | 0° / 90° | MG996R gate closed and open angles (SERVO_TEST_MAX_OPEN_MS = 10000 ms). |
| DEFAULT_SCALE_CALIBRATION | 420.0f | Initial HX711 factor before 2-point app calibration saves to NVS. |
| DISPENSE_TIMEOUT_MS | 30000 (30 s) | Maximum allowed gravimetric motor open time before emergency shutoff. |
| STALL_DETECT_MS / STALL_MIN_PROGRESS_G | 6000 (6 s) / 2.0f g | Closes gate if weight fails to climb by at least 2.0g within 6 seconds. |
| HOPPER_FULL_CM / HOPPER_EMPTY_CM | 5.0f cm / 30.0f cm | Ultrasonic distance thresholds mapped linearly to 100% and 0%. |
| BATTERY_DIVIDER_RATIO | 5.0f | Voltage divider multiplier ((40k + 10k) / 10k = 5.0) on GPIO 34. |
| BATTERY_FULL_VOLTS / SOLAR_CHARGING_VOLTS | 12.8f V / 12.6f V | 100% battery voltage and solar-charging detection threshold. |
| BATTERY_LOW_VOLTS / BATTERY_CUTOFF_VOLTS | 11.5f V / 11.2f V | Low-battery warning (11.5V) and hard servo lockout (11.2V = 0%). |
| TELEMETRY_INTERVAL_MS / COMMAND_POLL_MS | 10000 (10 s) / 1500 (1.5 s) | Cloud telemetry push cadence and remote command poll interval. |
| COMMAND_MAX_AGE_MS | 60000 (60 s) | Rejects stale cloud commands older than 60 seconds. |
| SCHEDULE_CHECK_MS / SCHEDULE_SYNC_MS | 5000 (5 s) / 30000 (30 s) | Local DS3231 schedule evaluation cadence and cloud schedule sync interval. |
| NTP_GMT_OFFSET_SEC | 28800 (UTC+8) | Philippine Standard Time (PST / Asia/Manila) NTP offset. |

## 13.4 Complete Local SoftAP HTTP REST Endpoints (`http://192.168.4.1:80`)

| HTTP Method | Endpoint | JSON Request Body | Purpose |
| --- | --- | --- | --- |
| GET | / | (None) | Renders standalone HTML mobile/desktop control portal in any browser. |
| GET | /api/telemetry | (None) | Returns current sensor telemetry JSON (feedLevelPercent, batteryVolts, status, etc.). |
| POST | /api/dispense | {"durationSeconds": 5, "targetGrams": 100.0} | Triggers timed or gravimetric dispense immediately. |
| POST | /api/stop | (None) | Immediately closes MG996R gate to 0° (aborted). |
| POST | /api/tare | (None) | Zeros HX711 load cell offset (0.0g). |
| POST | /api/calibrate | {"targetGrams": 500.0} | Computes new calFactor against reference weight and saves to NVS. |
| POST | /api/servo | {"servoAngle": 45} | Moves gate to 0°–90° for bench alignment (auto-closes after 10s). |
| POST | /api/sync-clock | {"epochSeconds": 1761840000} | Updates DS3231 RTC clock directly when operating 100% off-grid. |
| GET / POST | /api/schedules | [{...schedule objects...}] | Reads or overwrites up to 16 autonomous schedules in NVS flash. |
| GET | /api/logs | (None) | Returns JSON array of up to 25 recent FeedingLog entries from RAM ring buffer. |

## 13.5 Developer Build, Test, and Flash Commands

Run these commands in Windows PowerShell from the repository root (AutomaticDispensing/):

# 1. Verify Flutter Static Analysis (Developer) cd app; dart analyze lib/ test/ # 2. Run All 24 Automated Flutter Unit & Loopback Tests (Developer) cd app; flutter test --no-pub --timeout 30s # 3. Build Split-ABI Release Android APKs (Developer) cd app; flutter build apk --release --split-per-abi # 4. Build Progressive Web App Bundle for Hosting (Developer) cd app; flutter build web --release # 5. Compile ESP32 Firmware via PlatformIO (Developer) cd firmware; pio run # 6. Upload Firmware to ESP32 & Open 115200 Serial Monitor (Developer) cd firmware; pio run -t upload; pio device monitor -b 115200

# Chapter 14: Frequently Asked Questions (FAQ)

All answers below describe the exact, verified behavior of the repository's code:

## Q1: Does the machine require internet to feed my chickens?

No. Once your feeding schedules are saved onto the machine, the ESP32 stores them inside its permanent NVS flash memory and checks the time every 5 seconds using the battery-backed DS3231 Real-Time Clock. Your chickens will be fed on schedule even if the farm has zero internet for weeks. You only need internet if you want to monitor or trigger the feeder remotely from miles away via Firebase Cloud Mode.

## Q2: Can the machine work completely without Firebase?

Yes. Every ESP32 running this firmware simultaneously broadcasts a local Wi-Fi network named `SmartFeeder-AP` (password: chickens123) at `http://192.168.4.1`. On an Android phone, simply connect to SmartFeeder-AP and switch Settings → Hardware Transport to `Direct Wi-Fi HTTP (SoftAP 192.168.4.1)` (or open http://192.168.4.1 in any web browser) to view sensors, dispense feed, calibrate the scale, and set schedules without touching Firebase at all.

## Q3: Can I use my phone's mobile hotspot to give the ESP32 internet?

Yes, with one condition: Before uploading firmware, set WIFI_SSID and WIFI_PASSWORD in firmware/include/secrets.h to match your phone hotspot's exact Name and Password, and set your phone's hotspot frequency band to `2.4 GHz` ("Maximize Compatibility" on iPhone, or "2.4 GHz band" on Android). Whenever you turn on that hotspot near the coop, the ESP32 will automatically connect within 15 seconds and sync with Firebase!

## Q4: What happens if electricity is interrupted or the machine restarts?

Because the feeder runs on its own `12V` solar battery, utility grid brownouts/blackouts have zero effect on it.

If you manually switch the 12V battery off and back on:

setup() immediately forces the servo gate closed (`0°`) so feed cannot spill.

The ESP32 reloads your scale calibration (calFactor), all saved schedules (activeSchedules), and the last executed schedule slot (lastSchSlot) from NVS flash.

Any old cloud command sitting in Firebase is purged (purgeStaleCommandsOnBoot()) so the feeder never dispenses unexpectedly on power-up.

## Q5: What happens if the solar battery becomes low during rainy weather?

Two stages of protection are built in:

Warning Stage (`<= 11.5V` or below your `%` slider): Status changes to lowBattery, a red warning banner appears on the Dashboard, and your phone triggers a Low Solar Battery Alert notification while schedules still operate.

Hard Cutoff Stage (`< 11.2V` — `BATTERY_CUTOFF_VOLTS`): The firmware locks the `MG996R` servo motor closed (battery_cutoff_11v2). Preventing the high-current motor from firing below 11.2V protects the 12V battery from permanent deep-discharge damage and keeps the low-power ESP32 alive so it can still report its status until sunlight recharges the battery!

## Q6: Can I manually feed chickens anytime without messing up my daily schedules?

Yes! Triggering a manual feed (1s–30s) on the Dashboard runs an independent "Manual" dispense cycle and logs it separately in History & Logs. Your scheduled daily routines remain untouched and will still execute at their normal times.

## Q7: Can I schedule multiple feeding sessions per day?

Yes. You can configure up to `16` separate daily schedules (for example: 6:30 AM Morning Feeding, 11:30 AM Midday Feeding, 4:30 PM Afternoon Feeding). The app automatically prevents accidentally creating two schedules at the exact same hour and minute.

## Q8: How do I change the farm Wi-Fi SSID or password later?

(Developer / Technician Step) Open firmware/include/secrets.h on a PC with VS Code + PlatformIO, update WIFI_SSID and WIFI_PASSWORD to your new 2.4 GHz router credentials, connect the ESP32 via USB, and click PlatformIO: Upload (`→`). (Note: Your saved schedules and scale calibration in NVS are preserved!)

## Q9: How do I connect a replacement ESP32 if the board ever gets damaged?

Wire the new ESP32-WROOM-32 board using the exact pin table in Chapter 2.3 (remembering to check `5.0V` on `LM2596` first!).

Upload the same project firmware (pio run -t upload) with the same secrets.h file.

Because schedules live in Firebase /schedules, the replacement ESP32 will automatically download all your existing schedules from Firebase within `30 seconds` of connecting to Wi-Fi! Simply run a quick 2-point scale calibration in Settings → Diagnostics Suite for the new board.

## Q10: How do I reinstall the mobile application on a new phone?

Download app-arm64-v8a-release.apk onto the new Android phone (see Chapter 8.2) and open it. In Firebase Cloud Mode, all your existing cloud schedules, live telemetry, and historical feeding logs appear automatically as soon as the app connects!

## Q11: How do I update the firmware or mobile app?

Mobile App: Download the newer .apk file and tap Install—Android updates the app in place without erasing your local settings.

ESP32 Firmware: Pull or open the updated firmware/ folder in VS Code + PlatformIO, plug in USB, and click Upload (`→`).

## Q12: Can multiple phones monitor and control the same feeder machine?

Yes! In Firebase Cloud Mode, any number of farm staff phones installed with the app will receive real-time sensor updates, schedule changes, and feeding logs simultaneously. If Person A triggers a manual feed, Person B's phone immediately switches to "Dispensing in Progress..." and locks the manual feed button until the pour finishes so two people cannot double-feed at the same instant. (In off-grid `SmartFeeder-AP` mode, up to `4` local Wi-Fi devices can connect simultaneously per `WiFi.softAP(..., 1, 0, 4)`.)